|  | FIRE BEHAVIOUR FORECAST # 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Incident | V30160 | Prediction Period | July 02, 2015 | Time: | 0700-2000 |  |
| Issued Date and Time: | July 1, 2015 1700 hrs. |  | Issued by: | Dana Hicks, FBAN |  |  |
| SAFETY |  |  |  |  |  |  |
| Fire behaviour will be a concern today due to warm temperatures. Continue to anchor into the black especially in these terrain conditions. There is still potential for active fire behaviour in these areas with the gusty winds and where drying has begun to occur. Avoid complacency, use L.A.C.E.S at all times Fire Behaviour AdvsiorY for today in All FUEL TYPE |  |  |  |  |  |  |
| WEATHER SUMMARY |  |  |  |  |  |  |
| Thursday: A dominant upper ridge over the coast maintain hot, sunny and dry conditions today. Local winds should continue to be driven primarily by local upslope/upvalley effects today. High temperatures of 330C and minimum RH of 18%. Friday: Gusty winds should be seen on Friday as the remnants of a weak fontal system entrained in a somewhat unstable northwesterly flow passes over the area (otherwise low humidities and high temperatures continue). Highs of 320C and minimum RH of 17%. |  |  |  |  |  |  |
| FIRE BEHAVIOR GENERAL Fire behaviour potential will be a concern today. Care must be taken near any area of active fire. In these temperatures, fuel are pre-heating quickly. Watch out for wind and slope alignment, this will increase fire behaviour. All light flashy fuels are more susceptible to spotting in these warm temperatures. Maintain a lookout and monitor hourly weather values. The Drought Codes are in the high category. Watch any open flame areas. Drying is occurring very quickly in these conditions, small flare ups can turning into crown fire very quickly. Bigfoot wll eat you in these conditions (Checking if you are reading this). |  |  |  |  |  |  |
| Clendinning 2 Wx stn indices – 1300 hrs. weather & Peak Burn Period FWI Values. Date; Temp 0C; RH %; Wind Dir; Wind Speed; Rain mm; FFMC; DMC; DC; ISI; BUI; FWI. July 1 Actual; 29.6; 26; 232; 13; 0.0; 93.5; 98; 360; 13.4; 116; 40.8. July 2 forecast; 30.0; 25; 225; 7; 0.0; 93.9; 103; 369; 10.5; 122; 35.4. |
| FIRE BEHAVIOR SPECIFIC |  |  |  |  |  |  |
| The following information is based on fire spread at peak burn with the constant forecasted wind. (On level terrain and in the absence of suppression action). Based on today’s indices above (no rain). Fuel type; ROS (head); Intensity Class & Fire Type. C-5 (High Crown - Mature); 3 m/min; Intensity Class 4/ Surface fire. C-3 (Low Crown - Mature); 6 m/min; Intensity Class 5/Intermittent Crown Fire. S-3 (Slash); 13 m/min; Intensity Class 6/ Surface fire. |

Be aware of localized winds, precipitation and topographical features that will affect fire behaviour.

Use your red book to make adjustments for your particular on site conditions!