| A. PROJECT IDENTIFICATION |  |
| --- | --- |
| PROJECT ID AND UNIT ID: A075003 | LAND OR TENURE HOLDER: Kalesnikoff Lumber Co. Ltd. |
| LATITUDE/LONGITUDE: 49 22 27 – 117 31 39 | GEOGRAPHIC DESCRIPTION: Glade Creek |
| HIGHER-LEVEL PLAN(s): Kootenay Boundary Higher Level Plan Tarry’s Fire Protection Area - CWPP | MAP REFERENCE NUMBER: |

| B. Fuel Treatment PROJECT DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel Management OBJECTIVE: |  |  | This area has been identified as a priority treatment area within the Tarry’s Fire Protection Area – CWPP, RDCK electoral area I. To reduce the forest fuel hazards through a fuel modification treatments, so as to maintain biodiversity, enhance fire and forest health resilience from the perspective of climate change. |  |  |  |  |  |  |  |  |  |  |  |  |
| STRATEGIES: |  |  | Employ a seed tree silviculture system removing all dead and dying Lodgepole Pine while retaining fire resistant species such as Douglas-fir and Western Larch while maintaining wide crown spacing. Remove Specialty Forest Products concurrent with harvest to increase fibre utilization thereby reducing the amount of slash piles to be disposed of. Post-harvest reduce forest floor slash loading through the use of a machine rough bunch treatment. Regenerate the site with fire resistant species of a Douglas-fir, Western Larch and Western White Pine mix. |  |  |  |  |  |  |  |  |  |  |  |  |
| METHODS: Ground based mechanized harvest. Mechanized rough bunch treatment. Pile burning. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C. TREATMENT UNIT (TU) SUMMARY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TU | NET AREA (ha) |  |  | GROSS AREA (ha) | LEAVE AREAS (ha) |  | NP (ha) | NAR (ha) |  | TREATMENT REGIME |  |  |  | GENERAL DESCRIPTION |  |
| 1 | 17.6 |  |  | 21.2 | 1.5 |  | 0 | 17.6 |  | Ground system selective harvest with concurrent specialty forest product removal. Post-harvest mechanical rough-bunch treatment of slash. |  |  |  | Seed tree silviculture system. Post-harvest pile burn. |  |
| D. SITE CHARACTERISTICS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TU |  | CFFBPS FUEL TYPE |  |  |  | TIMBER TYPE |  |  | BGC SUBZONE, VARIANT & SITE ASSOC. |  | ELEVATION RANGE (m) | SLOPE POSITION | SLOPE RANGE (%) |  | ASPECT |
| 1 |  | C-4 |  |  |  | Lo36 Fd26 He21 Ce9 |  |  | ICHdw1 104 |  | 590 - 720 | Lower | 0-25 |  | Flat |

| E. SOIL CHARACTERISTICS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TU | SOIL TEXTURE | DUFF DEPTH (cm) | COARSE FRAGMENTS (%) | SOIL DISTURBANCE LIMIT (%) | SOIL HARZARD RATING |  |  |
|  |  |  |  |  | Compaction | Erosion | Displacement |
| 1 | L | 10 | 20 | 10 | H | M | L |

F. VALUES – FOREST AND RANGE PRACTICES ACT

View the accompanying Site Plan for A075003

| H. STAND AND STOCK TABLE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CURRENT STAND CONDITION |  |  |  | POST-HARVEST STAND CONDITION |  |  |
| Layer | Species % Composition | Stems/ha | Crown Closure % | Species % Composition | Stems/ha | Crown Closure % |
| Merch | Pl35Fd30Hw20Cw10(LwBlPwPy) | 623.5 | 65-70 | Fd65Lw30Cw5(PyPw) | 31.4 | 10 |
| Non- merch | Cw50Hw30Fd20 | 2000 | 15 | Cw50Hw30Fd20 | 140 | 5 |
| Post-harvest retention of merchantable: timber has been marked to leave at a ½ to 1 tree length spacing. Post-harvest retention of non-merchantable: stems has only been retained within the wildlife retention areas. 1.5ha is to be left intact for wildlife, biodiversity and visual screening of haul road. |  |  |  |  |  |  |

| I: TREATMENT SPECIFICATIONS |  |
| --- | --- |
| Timber harvest | Ground system selective harvest with concurrent specialty forest product removal. Merchantable stem retention is to focus on all marked to leave Fd, Lw, Pw and Py (31stems/ha). All dry grade 4 and grade 6 is to be harvested and removed from the site under specialty forest product authorization. Remaining harvest slash will be piled and burnt. |
| Coarse Woody debris >12.5cm | Where available retain a minimum of 4 pieces/ha up to a maximum of 150/ha distributed randomly throughout the area, avoiding accumulations (minimum size of 2m). Prefer retention of larger and decaying pieces lying flat to the ground. NOTE: Stumps due not contribute. |
| Fine fuels <12.5cm | Post-harvest fine surface fuels should not exceed the following: 15kg/m2 (15tonnes/ha) All fine surface fuels in excess of this target will be mechanically piled and burnt. |
| Debris disposal | Pile and burn will be the primary method of slash disposal. |

J. POST TREATMENT

EXPECTED VEGETATION RESPONSE: This site will be managed under timber production objectives where industry standard silviculture will apply.

| K. ADMINISTRATION |  |
| --- | --- |
| PREPARATION |  |
| FOREST PROFESSIONAL NAME (Printed) Tyler Hodgkinson | FOREST PROFESSIONAL SIGNATURE |
| MEMBER NUMBER 3984 | DATE November 23, 18 |