SKEENA SUSTAINABILITY ASSESSMENT FORUM

Forestry Working Group – Roads Subcommittee

“SSAF SC Road App” Supporting Guidebook

Date: July 10, 2023

Draft 1.0

Compiled by: 	Mike Birks, RPF  Mike.Birks@gov.bc.ca

MOF Skeena Stikine Natural Resource District

TABLE OF CONTENTS

1.	Introduction	4

2.	SSAF SC Road App - Attributes, Domain Names/ Definitions	5

Attribute (1) ROAD TYPE	5

Highway	5

MOT-Road	5

Resource Rd - Main	6

Resource Road - Branch	6

Resource Road - InBlock	7

Resource Road - Trail - Other	7

Recreation - Trail	8

Attribute (2) SURFACE TYPE	9

Paved		9

Gravel		9

Compacted/ Mineral Soil	10

Organics	10

Unknown	10

Attribute (3) STATUS CODE	11

Proposed	11

Existing		11

Daactivated	11

Overgrown	12

Rehabilitated	12

Attribute (4) ACCESS	13

2WD		13

4WD		13

ATV		14

Impassible	14

Proposed	14

Unknown	14

Attribute (5) STRUCTURE TYPE	15

Bridge		15

OBC - Open-Bottomed Culvert	15

WBC - Wood-Box Culvert	16

CMP - Corrugated Metal Pipe	16

CPP - Corrugated Plastic Pipe	17

Ford		17

Gate/ Barricade	18

Retaining Wall	18

Pulled		18

Attribute (6) STRUCTURE CONDITION	19

Functioning	19

Partially Functioning	19

Non-Functional	19

Attribute (7) SEDIMENT INPUT/ TRANSPORT POTENTIAL	20

Low		20

Moderate	20

High		21

Very High	21

ISSUES DESCRIPTIONS	22

3.	road terms	29

Fill Slope	29

Cut Slope	29

Road Prism	29

Beaver Stop	29

Ditch		30

Ditch Block	30

Sump		31

Flume		31

Inlet		31

Outlet		31

Armouring	32

- INTRODUCTION

This guidebook is intended to support the field use of the SSAF SC Road App for collecting and improving road data within the SSAF ESI area.

The guidebook is organized by the different road Attributes that are collected with the app (Road Type, Surface Type, Status Code, Access, Structure Type, Structure Condition, and Sediment Input/Transport Potential). For each Attribute there are a pre-established set of Domain Names which can be selected within the app. The guidebook provides definitions and example photos for the Domain Names.

Definitions and example photos are also provided some of the potential road issues (entered in the app under Comments) which individuals using the app may encounter while in the field, as well as for some common road terminology. Please note that the road issues listed in this document are not exhaustive of what could be encountered on the road.

Some Attributes and road issues may be difficult to identify without sufficient training. If an app user is unsure or uncomfortable choosing a Domain Name for a given Attribute please select Unknown as the default. If an app user sees a road issue that is not listed in this guidebook please make as descriptive a comment as you are able and take lots of pictures.

As this is Draft 1.0 of this guidebook, we are looking for user feedback on ways in which it could be improved.

Thank you.

- SSAF SC ROAD APP - ATTRIBUTES, DOMAIN NAMES/ DEFINITIONS

| Attribute (1): Domain Name: |  | Road Type Definition: |
| --- | --- | --- |
| Highway |  | Hwy 16, Hwy 37, Hwy 113, Hwy 62, Hwy 118, Hwy 35 |
| Figure 1 Highway 37 |  |  |
| MoT – Road |  | A road maintained by the Ministry of Transportation. May be difficult to distinguish from other roads in the field as they may be gravel or paved and in varying states of maintenance. To address this difficulty -a simplified spatial file of all MoT Roads as of 2023-07-14 (derived from WHSE_IMAGERY_AND_BASE_MAPS.MOT_ROAD_FEATURES_INVNTRY_SP) is made available to SSAF partners at the following FTP site location, for inclusion on field maps: https://www.for.gov.bc.ca/ftp/DSS/external/!publish/ESI/ |
| Resource Rd – Main |  | Generally 2 way haul. Public may very well be on these roads. Well maintained. |
| Figure 2 Typical signage at the front of a main Resource Road |  |  |
| Resource Rd – Branch |  | Generally 1 way haul. Roads that comes off of the main roads and lead to cutblocks. Road surface & access type variable - depends on time of year for construction, terrain, soil type etc. |
| Figure 3 Typical Branch FSR signage |  |  |
| Resource Rd – In Block |  | Roads that are constructed within Cutting Permit tenure. May or may not contribute to permanent access. |
| Figure 4 In block dirt road |  |  |
| Resource Rd – Trail/Other (eg. Bladed skidtrail) | Bladed trails, the old bark beetle access roads etc. |  |
| Figure 5 Bladed Skid Trail |  |  |
| Recreation - Trail |  | Recreational snowmobile/ ATV trails, established hiking trails |
| Figure 6 ATV/hiking trail |  |  |

| Attribute (2): Domain Name: | Surface Type Definition: |
| --- | --- |
| Paved | Includes pavement, seal coat, concrete, asphalt. |
| Figure 7 Paved rural road |  |
| Gravel |  |
| Figure 8 Maintained Gravel Road |  |

| Attribute (2): Domain Name: | Surface Type Definition: |
| --- | --- |
| Compacted/Mineral Soil | Examples would be block road with little to no gravel, bladed areas down to mineral soil |
| Figure 9 In block dirt road |  |
| Organics | Examples would be winter roads / areas where rig mats were removed. Roads that have grass growing on it. High likelihood these areas will be inaccessible after a season |
| Figure 10 A winter snow-fill road built over organics with minimal ground alteration. May be difficult to distinguish in the summer since it is mostly stumps and slash |  |
| Unknown | Indicates roads that may need a field visit to determine Surface Type |

| Attribute (3): Domain Name: | Status Code Definition: |
| --- | --- |
| Proposed | Proposed mainline or branch road, not in-block roads |
| Existing | Road was constructed & is not overgrown or rehabilitated. |
| Figure 11 Built and existing road |  |
| Deactivated | Per FPPR Deactivation definition. Road surface is still intact but structures have been removed, water bars in place & access limited |
| Figure 12 Grassy deactivated road. Cross ditch in foreground |  |

| Attribute (3): Domain Name: | Status Code Definition: |
| --- | --- |
| Overgrown | Road has continuous high-density (i.e. >30% cover distributed across full road width) woody brush / trees growing out of it. |
| Figure 13 Road covered with brush and impassible with any vehicle |  |
| Rehabilitated | Rehabilitated: road has had recontouring / decompaction work completed. - ie. Cannot drive a vehicle down. |
| Figure 14 Rehabilitated road with road surface decompacted and recontoured and slash scattered across road |  |

| Attribute (4): Domain Name: | Access Definition: |
| --- | --- |
| 2WD | A hard surfaced (“paved”) or well-maintained gravel road capable of travel by a 2-wheel drive vehicle |
| Figure 15 Maintained resource road accessible by 2WD |  |
| 4WD | A gravel or dirt road capable of travel by a 4-wheel drive vehicle. May not be regularly maintained, can have shallow deactivation features present, can have low to moderate density brush present |
| Figure 16 Non-maintained road only accessible by 4WD |  |

| Attribute (4): Domain Name: | Access Definition: |
| --- | --- |
| ATV | A gravel or dirt road capable of ATV travel only. Can have moderately deep deactivation features present (e.g. pulled major culverts or tank traps), may have low density brush present throughout the road prism or be brushed in along sides restricting access to the road centerline |
| Figure 17 Overgrown road accessible by ATV |  |
| Impassible | A gravel or dirt road that is impassible to a motorized vehicle regardless of the season of use. Reasons may include presence of very deep deactivation features (e.g. pulled bridges), piling of stumps within the road prism and ROW for >100 meters, and/or road is overgrown or rehabilitated. An access-controlled road (e.g. gated, temporary lock blocks) does not qualify as “impassible”. |
| Proposed | Road is planned but not yet constructed |
| Unknown | Accessibility is unknown; may need a field visit to determine. |

| Attribute (5): Domain Name: | Structure Type Definition: |
| --- | --- |
| Bridge | A temporary or permanent structure carrying a road above a stream or other opening |
| Figure 18 Bridge on a Resource Rd |  |
| Open-Bottomed Culvert/Arch | A drain pipe that allows for the natural stream bed to remain intact beneath the road allowing for fish-friendly passage |
| Figure 19 Open bottom culvert with natural stream channel |  |

| Attribute (5): Domain Name: | Structure Type Definition: |
| --- | --- |
| Wood Box Culvert (Log Culvert) | A log structure less than 6m long carrying a road above a stream or other opening (A log structure greater than 6m is a log bridge) |
| Figure 20 Illustrations showing the construction of a wood box culvert |  |
| CMP | Corrugated Metal Pipe. A metal drain pipe covered with soil and lying beneath the road (embedded in the road) surface used to carry either stream or ditch water from one side of the road to the other |
| Figure 21 Corrugated Metal Pipes |  |

| Attribute (5): Domain Name: | Structure Type Definition: |
| --- | --- |
| CPP | Corrugated Plastic Pipe. A plastic drain pipe covered with soil and lying beneath the road (embedded in the road) surface used to carry either stream or ditch water from one side of the road to the other |
| Figure 22 Corrugated Plastic Pipes |  |
| Ford | A dip in a road constructed to facilitate crossing a stream |
| Figure 23 Ford crossing through a stream |  |

| Attribute (5): Domain Name: | Structure Type Definition: |
| --- | --- |
| Gate/Barricade | Eg. A locked structure blocking access on the road; lock blocks blocking the road |
| Figure 24 Gate blocking access to road |  |
| Retaining Wall | A wall intended to hold soil in place and/or to keep soil from spilling onto the road |
| Figure 25 Retaining wall built on low side of road |  |
| Pulled | A point in the road where a structure has been removed |
| Figure 26 Road crossing where a crossing structure has been removed |  |

| Attribute (6): Domain Name: | Structure Condition Definition: |
| --- | --- |
| Functioning | Structure is working as intended with no adverse impacts to water or road and no need for repairs or maintenance. Structure is allowing for the unobstructed movement of water at full flow and is not causing sedimentation of a waterbody or negatively affecting the integrity of the road (eg. no erosion, no unintended pooling of water around structure, no damage to structure, no damage to road) Eg. A properly sized culvert that is passing all the water it is required to; no erosion around the inlet or outlet; no unintended pooling of water at inlet or plunge at outlet |
| Partially Functioning | Structure is not working at full capacity and has potential to deteriorate further without maintenance. Movement of water is partly obstructed and issues (damage to structure, erosion, etc.) are beginning to be observed. Eg. A partially plugged culvert that is still able to pass enough water at some times of the year but maybe not during high flow periods. May be signs of water pooling at inlet and causing erosion around inlet. Is at risk of getting more plugged and causing more damage |
| Non-Functional | Structure not working as intended and is resulting in damage to water or road and requires significant repairs or replacement. Movement of water is obstructed to the extent that significant sedimentation and/or damage to the road is occurring, or is at risk of occurring. Eg. A culvert that is either entirely plugged or plugged enough that it can no longer pass enough water to prevent damage and/or sedimentation from occurring |

| Attribute (7): Domain Name: | Sediment Input/ Transport Potential Definition: |
| --- | --- |

Sedimentation: the movement of soil into a waterbody

Sedimentation potential does not mean what is happening at the moment when the site is visited but what could happen under the right conditions. For example, if a stream is dry or if it hasn’t rained recently there may not be any active sedimentation but imagine what would occur at the site when water is present. Is there an issue which would emerge under rainy conditions that doesn’t occur under dry conditions?

*Please Note: If you have not received the FREP Water Quality Effectiveness Evaluation training, or an equivalent training on sediment potential, please select Unknown as the default answer for this attribute.

| Low | Unlikely that sedimentation will occur as a result of what is observed. No maintenance action is required. |
| --- | --- |
| Figure 27 Armouring around outlet of culvert creating low sedimentation potential by giving the water a clean surface to exit the culvert onto |  |
| Moderate | Some concerns noted and likely that a small amount of sedimentation will occur. Maintenance is not immediately required. |
| Figure 28 Soil being lost from the road at the edge of the bridge moving down towards stream. It may or may not end up in the stream. |  |

| Attribute (7): Domain Name: | Sediment Input/ Transport Potential Definition: |
| --- | --- |
| High | Likely that a large amount of sedimentation will occur and requires maintenance. |
| Figure 29 The drop at the outlet of the culvert will cause water to pick up and transport sediment downhill, into a stream if there is one there |  |
| Very High | Extremely likely that a large amount of sedimentation will occur. Major water quality problems exist at a landscape level and immediate action is required. |
| Figure 30 A long continuous ditch line drains directly into a stream |  |

| Issue Descriptions Examples of issues frequently encountered during road inspections. - Please Note that this is not a comprehensive list of potential road issues |  |
| --- | --- |
| Poor Ditching | Ditch no longer transporting water to desired location (eg. Into a culvert) |
| Figure 31 Stagnant water not ditched to the inlet of the culvert. Ditch block behind culvert. |  |
| Plugged Beaver Stop | Beaver stop becoming clogged with debris and causing (or in danger of causing) a reduction in culvert functioning |
| Figure 32 Beaver stop on outlet of culvert mostly plugged by beaver |  |

| Flume Detached from Culvert | Flume has become separated from culvert outlet and is no longer transporting water to the desired location |
| --- | --- |
| Figure 33 Part of the flume has become detached and erosion is occurring |  |
| Crushed/Collapsed Culvert | A culvert which has been crushed/collapsed at any point along its length and no longer capable of passing the intended amount of water |
| Figure 34 Partially collapsed culvert |  |

| Perched Culvert | Where the culvert outlet hangs above the level of the stream bottom creating a waterfall-like plunge of water. Creates a barrier for upstream movement of fish on fish streams. Excessive plunge on non-fish streams can also |
| --- | --- |
| Figure 35 Perched culvert with water dropping onto fill slope Figure 36 Perched culvert with plunge pool on a stream |  |
| Plugged Culvert | A culvert which has become filled with sediment or debris and is no longer capable of passing enough water |
| Figure 37 Culvert plugged with sediment |  |

| Piping Culvert | Where water is passing through the road prism around/below the culvert rather, or in addition to, through the culvert |
| --- | --- |
| Figure 38 Water eroding and passing around culvert rather than through it |  |
| Collapsed/Rotten Wood Box Culvert | Logs from the Wood Box Culvert becoming very soft or broken |
| Slide | A downslope movement of soil or rock. Generally either beginning at the road and travelling down or beginning above the road resulting in material landing on and covering road. Often a rapid occurrence |
| Figure 39 Slide beginning to accumulate material on road Figure 40 Slide beginning at the road |  |

| Slump | A mass of earth moves down a slope a short distance (rather than sliding all the way down). Often caused by saturation, undercutting, or overloading of the fill slope |
| --- | --- |
| Figure 41 Fill slope slump |  |
| Surface Erosion | Occurs where water remains on road surface rather than being shed to the side, causing water to drain improperly. Results in channels being washed out on the road surface and sediment transport down roads rather than in ditches |
| Figure 42 Surface erosion on road |  |
| Tension Crack | A crack running down the length of a road. May indicate unstable soil downslope of road. |
| Figure 43 Tension cracks running along the edge of a road |  |

| Tilting Trees Below Road | May indicate slumping downslope of road. As soil moves downward trees become tilted as their roots are shifted |
| --- | --- |
| Figure 44 Tilting tree below road |  |
| Sediment Accumulation on Bridge Deck | Sediment from road surface flowing onto bridge deck with a high likelihood of entering stream. May occur where bridge is located at the bottom of a hill |
| Figure 45 Sediment accumulating on bridge |  |

| Loss of Fill on Bridge | Road material from around the end of a bridge washing away towards stream. Bridge should hold back road material to prevent sediment from reaching stream |
| --- | --- |
| Figure 46 Loss of fill before bridge Figure 47 Significant loss of fill at the approach to the bridge |  |
| Non-Functioning Ditch Block | Ditch block no longer diverting water though culvert. Water may be continuing in ditch causing damage further down the ditch or redirected onto road surface |
| Non-Functioning Sump | Sump filled with sediment and no longer able to collect more sediment without being emptied |
| Figure 48 Filled sump, no space to catch more sediment unless it is emptied |  |
| Fan Influences |  |

- ROAD TERMS

| Fill Slope | the face of an embankment required to raise the desired road profile above the natural ground line. The downhill side of the road |
| --- | --- |
| Cut Slope | the face of an excavated bank required to lower the natural ground line to the desired road profile. The uphill side of the road |
| Road Prism | The width of road construction from the bottom of the fill slope to the top of the cut slope. |
| Figure 49 Width of road prism from top of cut to bottom of fill |  |
| Beaver Stop | A grate added to the inlet of a culvert intended to make it more difficult for a beaver to plug the culvert |
| Figure 50 Beaver stop on the inlet of a culvert |  |

| Ditch | A narrow channel running immediately parallel to the road intended to collect and transport water away from the road and/or sensitive ground |
| --- | --- |
| Figure 51 Recent ditching along road |  |
| Ditch Block | A dam in the ditch located on the downslope side of the inlet to a culvert, forcing water to divert from the ditch and through the culvert. Should be at least as high as the culvert but slightly lower than the road surface so that water remains in the ditch in case the culverts become damaged or plugged |
| Figure 52 Water properly ditched into the outlet of a culvert. Ditch block behind culvert. |  |

| Sump | An excavated hole intended to allow water to slow down and allow sediment to settle out before water reached a waterbody. Require regular maintenance to keep clear. |
| --- | --- |
| Figure 53 Water pooling in a sump at the end of a ditch |  |
| Flume | A half-culvert or pipe added to the outlet of a culvert intended to transport the water from the culvert further away from the culvert outlet before being deposited on the ground. Generally intended to transport water away from a fill slope to avoid the slope becoming saturated and potentially unstable |
| Figure 54 Flume attached to culvert outlet (culvert is hidden at the bottom of the photo) |  |
| Inlet | End of the culvert where water enters |
| Outlet | End of the culvert where water exits |

| Armouring | Use of some material (generally rocks) to provide erosion control around a structure or along a road or ditch |
| --- | --- |
| Figure 55 Culvert armoured with rock to prevent erosion |  |

Forest Service Road sign standards - Province of British Columbia (gov.bc.ca)

FSR sign - Outdoor Therapy

Managing resource roads - Rock to RoadRock to Road

KNP Logging Nearing Completion (kimberleynaturepark.ca)

The Top 5 Best Hikes In Montana (onlyinyourstate.com)

Malkow Lookout Trail Hike (near Smithers BC) - ihikebc.com

Gravel Roads - Jackson County, Oregon (jacksoncountyor.org)

Managing resource roads - Rock to RoadRock to Road

FOR Engineering Manual - Province of British Columbia (gov.bc.ca)

Deactivating forest roads to protect the surrounding environment - FPInnovations

Project Profile: Adams Groundhog Road Rehabilitation and Reforestation Project - Habitat Conservation Trust Foundation (hctf.ca)

SIR38 Bridge Planning Design and Construction (bcfpb.ca)

Slide 1 (wellsreserve.org)

FRE Layout (gov.bc.ca)

Acton Precast Concrete Limited

culvert pipes: meaning, types, uses - Studentlesson

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

Forest road retaining technologies on difficult slopes in Japan | Semantic Scholar

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

FREP_Extension_Note_12.pdf (gov.bc.ca)

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

Pipe Jacking solution solves culvert collapse – UKDN Waterflow

Crossing & Culvert Maintenance - streamhandbook.org

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

Lmh66_ch09.pdf (gov.bc.ca)

SIR47 - Resource Road Construction in Steep Terrain (bcfpb.ca)

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

FOR Engineering Manual - Province of British Columbia (gov.bc.ca)

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

NZ Forest Road Engineering Manual: Operators Guide • NZ Forest Road Engineering Manual: Operators Guide (nzfoa.org.nz)

All other photos provided by Ministry of Forests staff