DSS Annual Trap Tree Program – Locating Conventional Trap Tree (CTT) Sites	April 3, 2017

Objective:  Develop DSS-recommended locations for CTT sites along active road systems, to strategically protect mature Crown forest that is moderately to highly susceptible to spruce beetle (IBS).

- Trap trees are most effective at drawing IBS that are within 400m, but can attract IBS at distances up to 800m.

- Major licensees/ Community Forest/ Woodlots will be asked to confirm whether or not they are able to protect areas at risk in their operating areas by auctioning the DSS- identified trap tree sites along Road Permit roads, or sites of their own choosing (if not, DSS-identified CTT sites will be offered to SSS operators);

- BCTS will be asked a similar question with respect to FSR’s (if not, these will be offered to SSS operators);

- SSS operators will be given the opportunity to action CTT sites along non-status and non-forestry roads

DSS’ primary goal is to try to protect an annual population of most “at risk” IBS-susceptible forest in consideration of known current IBS population areas. Working concept is to use higher CTT site placement frequency/ closer site spacing (400m) where within 800m of 2016 DAOS points and areas (to effectively draw residual IBS from unactioned points and areas) and lower frequency/ wider spacing elsewhere (as IBS may or may not be present - trap trees are being used as a tool to strategically “sanitize” susceptible forest).

Steps for (GIS-based) Trap Tree Site location and Map production

1.  Select IBS LM to VH hazard class polygons (into new shapefile IBS_H-VH), dissolve all (call the dissolve file IBS_LM-VH_DSLV)

2. Select active roads (2WD). Clip to the extent of IBS_LM-VH_DSLV (call ACTRD_IBS-CLIP). Buffer 800m to either side (call ACTRD_IBS-CLIP_800mBUFF)

3. Buffer all 2016 DAOS IBS points and areas (2016_DAOS_PTS_FINAL;  2016_DAOS_POLY) by 800m, merge and dissolve (call this file 2016DAOS_800mBUFF)

5. Union 2016DAOS_800mBUFF   AND    IBS_LM-VH_DSLV    AND    ACTRD_IBS-CLIP_800mBUFF - call the new file IBS_BASE_2016

6. Add field “PRIORITY” to IBS_BASE_2016. Populate using the following codes and logic:

Update with “1” where all three files intersect.

“2” where the ACTRD_IBS.. file only intersects the DAOS_800m… file.

“3” where the ACTRD_IBS.. file only intersects the IBS_LM-VH… file.

Update remaining PRORITY with “4”.

7. Manually place trap tree site locations along the roads in ACTRD_IBS-CLIP, within IBS_BASE_2016. Build into a Arc pointfile (TrapTree.shp). Include Point ID’s.

Place CTT sites in Crown forest, avoiding the following areas:

  - Core Ecosystems

  - Special Management Zone 1

  - Wildlife Tree Retention Areas

  - Areas within 50m of a mapped water feature – river, stream, wetland or lake

  - Areas within 50m of a mapped Arch or Cultural Heritage feature – Arch site, SCHRIMP point or line, CMT cluster,

  - Slopes >60%

  - Rare and Endangered Ecosystems

Use the following CTT site placement frequency rules:

PRIORITY “1” areas – CTT site spacing should be 400m along 2WD rds in these polygons

PRIORITY “2” areas - 800m CTT site spacing along 2WD roads

PRIORITY “3” areas – 1000m CTT site spacing along 2WD roads

PRIORITY “4” areas – no CTT sites

Add a SUBPRI (sub-priority) attribute field to TrapTree.shp. Populate using the following codes and logic:

HIGH – CTT sites in PRIORITY “1” and “2” areas
MOD – CTT sites in PRIORITY “3” areas that are within a 3km buffer around High to Extreme IBS Hazard types
LOW – remaining CTT sites in PRIORITY “3” areas

8. Build maps.

a. Use the same MXD (and map template) as was used for the TACTIC plan

b. Keep same layers active, except turn off anything related to IBM

c. Add IBS_BASE_2016.

d. Add ACTRD_IBS-CLIP. Theme by Road Permit versus FSR versus Other.

e. Add TrapTree.shp. Theme by SUBPRI.

9. Generate PDF maps by Chart area (immediate priority Bulkley and Kispiox, secondary priority Cassiar). Post to the following location on DSS FTP site:  https://www.for.gov.bc.ca/ftp/DSS/external/!publish/Web/Forest%20Health%20Strategy%202015/5_Beetle%20Tactical%20Plans/Trap%20Tree%20Program/Operating%20Area%20Maps/