|  |  | BC Timber Sales Environmental Management System environmental OPERATING PROCEDURE ## EMS Incident Reporting and Investigation |  |
| --- | --- | --- | --- |
| Revised: April 1, 2015 EOP - 04 |  |  |  |
| # SCOPE/PURPOSE: The scope of this environmental operating procedure (EOP) applies to significant environmental events associated with activities/operations within the scope of BCTS EMS including; harvesting, silviculture, access and planning. # The purpose of this EOP is to outline BCTS staff procedures and responsibilities for EMS incident reporting and investigation. |  |  |  |
| # EMS INCIDENT REPORTING REQUIREMENTS: - Emergency Response Incidents: - Fire: Any uncontrolled fire related to BCTS activities - Spills: Any spill exceeding BCTS reportable levels or any amount spilled into or immediately adjacent to a stream, lake or running water. - Erosion/Landslide Events: Any emergency or potential emergency situation, including; abnormal soil movement or sedimentation creating a significant risk to the environment or public safety, movement or an imminent risk of movement of a volume of material greater than 250 m3, or disturbance through erosion processes of a land area greater than 0.25 ha. - Potential Non-Compliance: - In the opinion of the person reporting, legislation and regulation has been violated and there may be an agency investigation to determine fact and possible enforcement action. - Significant Non-conformance: - An occurrence or event that has or will likely result in a negative environmental impact and cannot be immediately rectified. - When the EMS program has been severely compromised and or a “Notice to Comply” has been issued at the discretion of local management. This includes repeated non-conformances that may become significant.; #### Reportable Levels. #### Substances; #### BCTS; #### EMBC. Antifreeze; 5 litres; 5 litres. Diesel fuel; 25 litres; 100 litres. Gasoline (auto & saw); 25 litres; 100 litres. Greases; 25 litres; 100 litres. Hydraulic Oil; 25 litres; 100 litres. Lubricating Oils; 25 litres; 100 litres. Methyl Hydrate; 5 litres; 5 litres. Paints & Paint Thinners; 25 litres; 100 litres. Solvents; 25 litres; 100 litres. Pesticides; 1 kg or 1 litre; 1 kg or 1 litre. Explosives; Any; Any.. |
|  | PROCEDURE: |  | # Responsibility |
| Reporting | Review and follow-up any reported or identified EMS incidents as defined above. |  |  |
|  | Field Tech receives spill report and reports incident to their supervisor as soon as possible. Complete Part A CHK-009 and/or enter incident into EMS Issue Tracking System (ITS). Refer to the Certification Module Supplementary User Guide for additional information. |  |  |
|  | If Emergency Response Incident, ensure appropriate agencies have been notified and ensure the reporting Licensee / Permittee / Contractor or BCTS staff member has followed the requirements of the eERP such as: initial response action, agency reporting and follow-up. |  |  |
| Investigation | Determine if further investigation is necessary from reported information. If a decision is made not to conduct an incident investigation, provide rationale on EMS Incident Report Form CHK-009 or in ITS. Rationale must be supported by supervisor or CSO. Incidents that don’t have a significant impact on the environment, (i.e. Timber Transport and Marking) or incidents that can be immediately rectified (i.e. industrial waste left on site or a blocked cross drain culvert) don’t require root cause investigation but one may be initiated if it is a repeat or chronic issue. |  |  |
|  | Determine who should conduct the investigation based on severity of event, risk and consequence. - Level 1: Includes most reported EMS incidents, investigation usually conducted by BCTS staff that are administering and or supervising the applicable project. - Level 2: This type of investigation is more complex requiring additional time, experienced personnel, knowledge, or resources to complete, and may involve BCTS staff and/or external experts. Usually associated with incidents when there is a significant impact to environment, social and economic values. |  |  |
|  | Conduct Incident Investigation to determine root cause and assess: - Immediate actions undertaken and proposed to be undertaken to address impacts. - Details of the incident including a timeline, contributing factors and root cause(s). - Adequacy of response measures, including training, equipment capabilities, procedures, etc. - Licensee, Permittee, Contractor, and company conformance to procedures or instructions. - Control of impact, including timeliness of response. |  |  |
|  | Develop recommendations for corrections and corrective actions and appropriate action plans identifying: - Immediate actions to be undertaken to address impacts. - Additional actions required to address root cause(s). - Responsibilities, timelines, and verification of completion of actions. |  |  |
|  | Ensure details of investigation are documented in the EMS Issue Tracking System (ITS) and/or complete Part B of the Incident Report Form CHK-009 and file appropriately. Refer to the Certification Module Supplementary User Guide for additional information. |  |  |
|  | Obtain Timber Sales Manager or designate approval, document on CHK-009 or in ITS. |  |  |
|  | Implement corrections and corrective actions, follow-up, update and close ITS and action plans. |  |  |