- Creation of a somewhat seamless forest cover layer for the province:

- Most of the data came from 2002 VRI data supplied by James Wang

- TFL data came from the timber reallocation project

- Vegetation information from the State of the Forest (circa 2001 and supplied by Stephen Sutherland) was used to fill gaps in the TFL data and the VRI supplied by James Wang

- Unfortunately the TFL data did not have volumes for each of the typical 6 levels recorded in VRI forest cover.  It did, however, have total volume and the percents for each of the 6 levels.  So I apportioned the total volume amongst the 6 species levels depending on the percent coverage.

- The resulting “seamless” forest cover polygon layers was converted to separate rasters for each of the 6 species, percent and volume levels (a total of 18 raster layers) all with a raster cell size of 100m x 100m ( 1 ha).  Separate rasters were also created for age and site index –also with a 1 ha cell size.

- Creation of  a provincial timber harvesting land base layer:

  - Again a “somewhat seamless” layer was created but this time from a sole source –the timber reallocation project.  THLB information was missing from some small TFLs.

  - I didn’t want to handle THLB Inclusion/Contribution Factors so a cell was considered THLB if the contribution (a value between 0 and 1, where 0 means not included in the THLB and 1 means fully included in the) was less than a random number between 0 and 1.  So for example, a cell with a THLB inclusion factor of 0.8 had a 80% chance of becoming fully included in the THLB.

- Create Aerial Overview Forest Health Factor layers:

  - Aerial Overview polygon data and spot data were processed differently before being combined into a single raster of severity for each Forest Health Factor for each year.

  - Each year of the Aerial Overview polygon data was simply converted to a separate raster of severity for each forest health factor.

  - Spot locations were converted to circle polygons where the circle area reflected the areas recorded in the Aerial Overview spot data (i.e., 0.25ha, 0.5ha, etc).  The resulting circle polygons were converted to raster and given a ‘severe’ rating.

  - For each year of the aerial overview, the spot-source and polygon-source raster layers for a particular forest health factor were combined into a single raster, with spot-source data taking precedence over the polygon-source data.

- Create logging history layer:

  - A provincial raster layer of logging history was created from a polygon layer of logging history provided by Iaian McDougall.

- Estimate non recovered losses:

  - Based on the advice of Tim Ebata, a decision was made to only assess the non recovered losses caused by the forest health factors listed in the table below because they represented the largest impacts on timber supply.  Also included in the list below is the tree species impacted by each forest health factor.

| Disturbance Description (forest health factor) | Disturbance Code | Affected Tree Species Code |
| --- | --- | --- |
| Fire | NB | All Species |
| Drought | ND | All Species |
| Flooding | NF | All Species |
| Mountain Pine Beetle | IBM | Pl, Pw, Pa, Pf |
| Western Balsam Bark Beetle | IBB | Bl |
| Spruce Beetle | IBS | Sx |
| Douglas-fir Beetle | IBD | Fd |
| Fir engraver Beetle | IBF | Ba, Bl |
| Engraver Beetle (Ips species) | IBI | Pl |
| Lodgepole Pine Beetle | IBL | Pl |
| Twig Beetle | IBP | Pl |
| Red Turpentine Beetle | IBT | Pl |
| Western Pine Beetle | IBW | Py |
| Douglas-fir Tussock Moth | IDT | Fd |
| Yellow cedar decline | NCY | Yc |
| Unknown Beetle | IB | All conifers |

  - Estimating the volume killed by each forest health factor was done iteratively for each year of the Aerial Overview.

  - Starting in 1999:

    - The vegetation volumes were first reduced to zero in areas overlapping with forest harvesting that occurred in, or prior to, the year being processed.

    - Starting with the volume layers created in the prior step, each forest health factor (FHF) listed in the above table was in turn overlaid (in the order listed in the table) with the vegetation volume, species, and percent information to:

      - Reduce the volume of the affected tree species depending on the mapped FHF severity rating (see table below).

      - Adjust the percent-cover for each species level to reflect the volume loss attributed to a particular FHF.

      - Create a raster of volume loss (volume killed) attributed to a particular FHF in a particular year.

| Severity Rating | Percent Volume Reduction |
| --- | --- |
| Very Severe | 75 |
| Severe | 30 |
| Moderate | 15 |
| Low | 5 |
| Endemic | 0.5 |

    - The updated species volume and percent raster layers were subsequently used as input when processing the next FHF.

    - Once all FHF were processed for a particular year, the resulting vegetation volume, species, and percent raster layers (adjusted for volume loss) were used as the input vegetation information for the subsequent year to be processed.

  - The resulting volume loss/kill rasters for each FHF and year were subsequently overlaid with the TSA boundaries and summarized by year, FHF, TSA and volume loss.

  - To estimate the NRL, the areas harvested were removed from the volume loss/kill rasters to create a separate set of NRL volume rasters.  These NRL volume rasters were overlaid with the TSA boundaries and summarized by year, FHF, TSA and NRL volume loss.

  - Starting with the 1999-2015 NRL estimate, the NRL estimates for mountain pine beetle (disturbance code IBM) were replaced with the output from the Provincial Mountain Pine Beetle Spread Model (aka BCMPB; www.for.gov.bc.ca/hre/bcmpb).