UPDATE: South Coast 7Dec2010  11:30PM

Flood Warning: Tsolum River

Flood Watch: Browns River and Cruikshank River

High Streamflow Advisory:

- West Vancouver Island

- East Vancouver Island

- Howe Sound

- Lower Mainland

The BC River Forecast Centre is upgrading:

- the Flood Watch to a Flood Warning for the Tsolum River;

- the High Streamflow Advisory to a Flood Watch for the Cruikshank River,  and maintaining the Flood Watch for the Browns River;

- and maintaining the High Streamflow Advisory for the South Coast, including west and east Vancouver Island, Howe Sound and the Lower Mainland.

Environment Canada has continued the Rainfall Warning for the areas listed above.  A slow moving frontal system is forecast to deliver another 20 to 60 mm of rain for East Vancouver Island, Metro Vancouver and Howe Sound overnight. Western and Inland Vancouver Island are forecast to receive another 50 to 70 mm of rain overnight and into Wednesday morning.

The Tsolum River near Courtenay (WSC-08HB011) is currently at 217 m3/s, just below a 5 year return period discharge of 219 m3/s, with a rate of rise of 15 cm/hour.

The Browns River near Courtenay (08HB025) is currently at 73 m3/s, and is approaching a 2 year return period discharge of 112 m3/s, with a rate of rise of 5 cm/hour.

The Cruikshank River (08HB074) is currently at 168 m3/s and is approaching a 2 year return period of 252 m3/s, with a rate of rise of 18 cm/hour.

24 hour rainfall totals range from 46 mm at the North Courtenay MoT station, to 234 mm at Tofino Creek (08HB086), with most areas receiving accumulations of 60-80 mm. Current precipitation rates are steady at 3-7 mm/hour.

Rivers elsewhere on the South Coast remain well below levels of concern, and aren’t expected to reach flood stages overnight.

Freezing elevations remain near treeline, at approximately 1400 m over most of the South Coast, and are expected to remain stationary overnight, and beginning to lower by Wednesday morning.  Any remaining low elevation snow remaining on the ground is expected to melt, contributing to runoff.

Given the current weather forecast, river levels are expected to continue to rise overnight, peaking early Wednesday morning, before beginning to recede.  Localized flooding in low-lying areas may occur.

The River Forecast Centre will continue to monitor conditions and will provide an update Wednesday morning, or earlier if conditions warrant.