UPDATE – Flood Watch: Southeast Peace Region

High Streamflow Advisory: Southwest Peace Region

Issued: 11July2011 3:45 PM

The BC River Forecast Centre is downgrading the Flood Warning to a Flood Watch for:

- Southeast Peace Region

The BC River Forecast Centre is downgrading the Flood Warning to a High Streamflow Advisory for:

- Southwest Peace Region

Rainfall has eased in the Peace Region and river levels in most southwest areas (Chetwynd) have started to decline.  The Pine River at East Pine (WSC 08FB001) has fallen from its peak of 6.25 m early Sunday morning down to 5.2 m at mid-day today, which is just above a 2-year return period water level.  The Moberly River near Fort St. John (WSC 08FB008) has plateaued at a water level of 3.3 m since yesterday due to the storage of water in Moberly Lake, and will start to recede over the next few days.  Due to the higher amounts of rainfall received in the eastern Peace Region (Dawson Creek) and the flatter topography of the area, river levels may remain elevated and will take longer to recede over the next few days.

The Environment Canada forecast is for unsettled conditions with 5-10 mm of light precipitation today, and there is the potential for additional precipitation in the southeast by late Wednesday.   The River Forecast Centre will continue to monitor conditions and will provide an update as conditions warrant. Current streamflow information is available from the River Forecast Centre website: http://bcrfc.env.gov.bc.ca/.