UPDATE: High Streamflow Advisory – Lower Fraser River

Issued: 23June2012  3:00PM

The BC River Forecast Centre is maintaining a High Streamflow Advisory for:

- Lower Fraser River

As of 8:00AM on Friday, the Fraser River at Hope (08MF005) is flowing at 11,750 m3/s, and has dropped slightly from a peak of 11,900 m3/s on Friday. River levels at Hope are expected to remain elevated (>11,500 m3/s) through the weekend and early next week.

Water is currently working its way through the lower reaches of the Fraser Valley (Chilliwack downstream to the ocean. As a result of increases in local inflows (including the Harrison River) and channel hydraulics, river levels are expected to continue to rise through the weekend through the Fraser Valley and reach peak levels on Sunday and Monday. For complete forecasts of the lower Fraser River water levels and timing of daily peak levels, see the River Forecast Centre website: http://bcrfc.env.gov.bc.ca/bulletins/Freshet2012/WL_Prediction.pdf

The Fraser River at Mission (08MH024) is currently at 6.37 m and is expected to rise up to 6.50 m on Sunday to Monday.

The Fraser River at Whonnock (08MH044) near Langley is currently at 5.09 m, and is expected to rise to 5.14 m on Sunday to Monday.

As of Saturday morning, a low pressure system was bringing rain across much of southern British Columbia. This is forecast to continue to deliver more rain through Sunday, with the heaviest amounts expected in the Thompson River basin (Shuswap/South Thompson and North Thompson Rivers). With strong influences from large lake systems in the Thompson River (eg. Shuswap, Adams, Kamloops), the river responds to rainfall with a longer, attenuated, rise in river levels over a period of many days. In the Upper Fraser River (upstream of Prince George), the watershed is mountainous and runoff from rainfall rapidly moves downstream (creating significant “pulses” of water that travel down the river, such as what is being observed through the lower Fraser River this weekend).

River levels through the Lower Fraser Valley are expected to drop slightly on Tuesday, and then begin rising again towards next weekend. The river is expected to see the potential for increases above current levels over the Friday-Monday period next week. The amount of rise will depend on where and how much rain falls over the next few days.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.