UPDATE: High Streamflow Advisory – Lower Fraser River

Issued: 23June2012  3:40PM

The BC River Forecast Centre is maintaining a High Streamflow Advisory for:

- Lower Fraser River

As of 8:00AM on Saturday, the Fraser River at Hope (08MF005) is flowing at 11,750 m3/s, and has dropped slightly from a peak of approximately 11,900 m3/s on Friday. River levels at Hope are expected to remain elevated (>11,500 m3/s) through the weekend and into early next week.

Water is currently working its way through the lower reaches of the Fraser Valley (Chilliwack downstream to the ocean). As a result of increases in local inflows (including the Harrison River) and channel hydraulics, river levels are expected to continue to rise through the weekend through the Fraser Valley and reach peak levels on Sunday and Monday. For complete forecasts of the lower Fraser River water levels and timing of daily peak levels, see the River Forecast Centre website: http://bcrfc.env.gov.bc.ca/bulletins/Freshet2012/WL_Prediction.pdf

The Fraser River at Mission (08MH024) is currently at 6.37 m and is expected to rise up to 6.50 m on Sunday into Monday.

The Fraser River at Whonnock (08MH044) near Langley is currently at 5.09 m, and is expected to rise to 5.14 m on Sunday into Monday.

As of Saturday morning, a low pressure system was bringing rain across much of southern British Columbia. This system is forecast to continue to deliver more rain through Sunday, with the heaviest amounts expected in the Thompson River basin (Shuswap, South Thompson and North Thompson Rivers). In the Upper Fraser River (upstream of Prince George), the watershed is mountainous and runoff from rainfall rapidly moves downstream, creating significant “pulses” of water that travel down the river, such as what is being observed through the lower Fraser River this weekend. On the other hand, with strong influences from the large lake systems in the Thompson River (e.g. Shuswap, Adams, Kamloops), the Fraser River responds to rainfall from those areas with a longer, attenuated, rise in river levels over a period of many days. This is what is expected to occur over the coming week.

River levels through the Lower Fraser Valley are expected to drop slightly on Tuesday, and then begin rising again towards next weekend. There is potential for increases above current levels over the Friday to Monday (June 29th – July 2nd) period next week. The amount of rise will depend on where and how much rain falls over the next few days.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.