UPDATE- Provincial Flood Forecast – Update on Warnings, Watches, Advisories

Issued: 25June2012 10:00AM

The following provincial flood watches, warnings and advisories remain as issued on 24June2012 @ 1:00PM:

NORTH INTERIOR (Skeena, Omineca, Peace)

- Bulkley-Skeena – Flood Watch

- Quesnel River – Flood Watch

- Fraser River (at Prince George and upstream, and downstream from Quesnel to Fraser Canyon) – Flood Watch

- Nechako River near Prince George – High Streamflow Advisory

SOUTH INTERIOR (Cariboo, Thompson, Okanagan, Kootenay)

- South Thompson River and Shuswap Lake – Flood Warning

- Shuswap River upstream and downstream of Mabel Lake (including around Enderby and Lumby) – Flood Warning

- Thompson River (including Kamloops area and Kamloops Lake)- Flood Watch

- Upper Columbia Region – Flood Watch

- Kicking Horse River – Flood Watch

- Kootenay – Flood Watch

- North Thompson River and tributaries – Flood Watch

- Chilcotin – High Streamflow Advisory

- Okanagan – ENDED

COAST

- Bella Coola River – High Streamflow Advisory (DOWNGRADED)

Rivers have been responding to rapid snowmelt and wet conditions over the past 2-3 weeks. Heavy rainfall occurred through the Okanagan, Shuswap, Chilcotin and Kootenay regions over the weekend has led to significant river level rises in these areas.  A summary of current conditions:

- Fraser River at South Fort George (08KE018) is currently at 9.25 m and rising. The river is expected to continue to rise today and into tomorrow. The current forecast is for levels to reach 9.7 m,

- The Bulkley River at Smithers (08EE005) is currently at a level of 5.1  m with flows of 970 m3/s. Additional rises of 5-10 cm are possible through Monday. Additional rises are possible if rainfall is heavy in the watershed over the next few days.

- The Skeena River at Usk (08EF001) is currently at 6170 m3/s (5-year flow). and has risen nearly 1000 m3/s (or 1 metre) since yesterday. The river is near peak level and is expected to peak today with additional rises of 200-300 m3/s (20-30 cm). Additional rises are possible if rainfall is heavy in the watershed over the next few days.

- Shuswap Lake at Salmon Arm (08LE070) is at 349.32 m (30-year level). Additional rises of 5-10 cm are expected through Tuesday.

- South Thompson River at Chase (08LE031) is at 1400 m3/s (30-year flow). The river is expected to see additional rises of 50-100 m3/s through Tuesday.

- The Thompson River at Overlander Bridge in Kamloops is currently at 343.84 m Additional rises to 344.3-344.4 m are expected through Wednesday, but may be higher if additional rain occurs.

- Slocan River near Crescent Valley (08NJ013) is currently at 560 m3/s (5 year flow). The river is expected continue to drop into tomorrow.

- The Kicking Horse River at Golden (08NA006) is currently at 346 m3/s (approaching 20-year flow) and is near flood stage. The river is expected to drop into tomorrow, unless significant additional rainfall occurs

- The Kootenay River at Kootenay Crossing (08NF001) is reaching peak levels (approximately 47 m3/s or a 20-30 year flow). Additional rises are possible if more rain impacts the area.

- Mission Creek near East Kelowna (08NM116) has continued to drop and is now below a  2-year flow level.

Conditions in the Lower Fraser River are provided in a separate advisory.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.