UPDATE: Flood Warning: Upper Fraser River

Issued: 19June2012  7:30AM

The BC River Forecast Centre is maintaining a Flood Warning for:

- Fraser River (at Prince George and upstream, and downstream from Quesnel to Fraser Canyon)

River levels have been rising over the past 2 days through the upper Fraser River. The McGregor River at Lower Canyon (08KA004) peaked late-Sunday at 1800 m3/s (20-year flow) and has since dropped to approximately 1300 m3/s.  The Fraser River at Shelley (08KB001) is still rising at approximately 20 m3/s per hour, and the rate of rise has eased since yesterday. The Fraser at Shelley is currently at 4470 m3/s (20-year flow) and is forecast to peak late this morning or early afternoon at 4550 m3/s (approximate additional rise of 10-20 cm above current levels). The Fraser River at South Fort George is currently at 9.80 m and rising at 2 cm/hour. The Fraser River at South Fort George is forecast to peak at 9.95-10.20 m later on Tuesday evening. The last time river levels exceeded 9.9 m was in 1972.

River levels are expected to be at flood stage downstream of Prince George through to the Fraser Canyon, starting on Wednesday and extending through to Thursday and Friday.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.