UPDATE: Flood Warning: Liard River

Issued: 08June2012  11:45AM

The BC River Forecast Centre is upgrading the advisory to a Flood Warning for:

- Liard River at Upper Crossing (near Lower Post)

- Liard River at Lower Crossing

Heavy rain has impacted the area over the past several days. The Liard River has been rising sharply in response to this rainfall, and on-going snowmelt.  As of 6AM on Friday, the Liard River at Upper Crossing (10AA001) was at 2500 m3/s, and gauge height of 6.387m, and rising 3 cm/hour (5 year flow). The river is expected to peak at up to 3000 m3/s, or gauge height of 6.8-7.0 m, on Saturday, June 9th. This would approach a flood of record for this station (51 year record).

The Liard River at Lower Crossing is currently at 6075 m3/s and rising at 70 m3/s. The river is expected to peak tomorrow at approximately 7000-7500 m3/s, or a 10-20 year flow level.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.