High Streamflow Advisory: Central Coast

ISSUED: 05October2013  11:30AM

The BC River Forecast Centre is issuing a High Streamflow Advisory for the Central Coast including:

- Kingcome River

Central and northern coastal British Columbia is currently being impacted by a number of Pacific storm systems. Heavy rainfall over the past 48 hours has led to elevated river levels throughout the region. Rain has eased today through the central coast, and this is allowing for river levels to ease slightly.

The Kingcome River below Atlazi River (WSC 08GF009) has dropped from a high overnight level of 7.4m, down to a current level of 6.3m.

Another system of heavy rainfall is expected later on Sunday and overnight into Monday. Rainfall amounts in the 40-60mm range are possible through the central coast. Additional rises are expected on the Kingcome River overnight in response to this additional rainfall. River levels are expected to peak overnight or early Monday, with forecasted levels at the Water Survey of Canada gauge (08GF009) reaching the 7.0-8.0m range. The amount of rise will depend on rainfall rates later today and overnight, and higher levels are possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.