Flood Watch: Bulkley River

UPDATED: 14 May 2015 2:15 PM

The BC River Forecast Centre is issuing a Flood Watch for:

- Bulkley River

The Bulkley River has experienced rising flows over the past several days due to warmer weather and increased snow melt rates. The Bulkley River at Quick (Water Survey of Canada gauge 08EE004) is currently flowing at 670 m3/s (5-year return period flow) with a gauge height of 2.21 m. The Bulkley River near Smithers (WSC 08EE005) is flowing at 890 m3/s, with a gauge height of 4.98 m.

High pressure has settled over northern British Columbia, Yukon and Northwest Territories and is bringing prolonged warm weather through the north interior of BC. Snow melt rates have been elevated over the past several days, with melt rates on 25-30 mm/day being observed at the Lu Lake snow pillow (4B15P) over the past 5 days.

Temperatures are expected to remain elevated through the weekend. River levels continue to rise in response to rapid snow melt, and are expected to continue to rise into the weekend. The Bulkley River at Quick (08EE004) is forecast to rise to 800 m3/s (gauge height 2.5 m) through Sunday, while the Bulkley at Smithers is expected to see a rise to 1000-1100 m3/s, or an additional rise of 20-30 cm at the gauge (5.2-5.3 m). Flooding in low-lying areas is expected, and similar high flow conditions are expected on tributary rivers through the Smithers area.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water.  Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.