FLOOD WATCH: Nechako River

UPDATED: 15 May 2015 10:00 AM

The BC River Forecast Centre is upgrading the High Streamflow Advisory to a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

The Nautley River near Fort Fraser (08JB003) is estimated to be flowing at 226 m3/s (gauge water level is 2.75 m) which is above the 20-year return period flow. The Nechako River at Vanderhoof (08JC001) is flowing at 530 m3/s (gauge water level is 4.71 m), which is above the 10-year return period flow. The Stellako River at Glenannan (08JB002) is flowing at 157 m3/s and gauge water level is 3.22 m.

This week’s warm temperatures have increased snowmelt to approximately 30 to 40 mm per day at high elevation areas of the basin. This has caused increased flows throughout the region. Warm and dry conditions are expected to continue until next Tuesday.

Discharge from the Nechako Reservoir has been decreased to 270 m3/s this morning with Skins Lake Spillway continuing to be regulated to minimize the flood risk to Vanderhoof.

Given the current conditions and the weather forecast for the next five days, flows on the Nautley River are expected to continue to gradually increase and reach 235 to 245 m3/s by Monday and peak by late next week. The Nechako River at Vanderhoof is forecast to reach 540 to 550 m3/s by Monday. The Stellako River at Glenannan is expected to see a rise of 5-10 cm over the weekend, and similar rises may be expected on Francois Lake.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 793 m3/s (gauge water level is 4.33 m) and are expected to reach 860 m3/s by Monday. Flows on the Fraser River at South Fort George (WSC 08KE018) are expected to rise but remain below levels of concern.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water.  Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.