FLOOD WATCH: Nechako River

UPDATED: 18 May 2015 8:00 AM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

The Nautley River near Fort Fraser (08JB003) is currently estimated to be flowing at 245 m3/s (gauge water level is 2.83 m) which is above the 20-year return period flow. The Nechako River at Vanderhoof (08JC001) is flowing at 551 m3/s (gauge water level is 4.82 m), which is above the 10-year return period flow. The Stellako River at Glenannan (08JB002) is flowing at 167 m3/s and gauge water level is 3.27 m.

A modest easing of temperatures over the weekend has led to a slight decrease in snow melt rates. Observed melt rates have been in the 20 to 30 mm per day at snow pillows in the region. Temperatures are expected to gradually rise through the week, with temperatures in Vanderhoof expected to rise from 21˚C today, to 26˚C by the end of the week. Snow melt rates are expected to rise with increasing temperatures.

Given the current conditions and the weather forecast for the next five days, flows on the Nautley River are expected to continue to gradually increase through the week, and is forecast to reach approximately 260-265 m3/s by Thursday. With snow conditions in the watershed, the Nautley River is expected to reach peak levels later this week. The Nechako River at Vanderhoof is forecast to see modest additional rises through the week, with the potential for 4-8 cm additional rise through Thursday (flows reaching approximately 560 m3/s). The Stellako River at Glenannan is expected to see continued rises of 2 cm/day through mid-week, and similar rises may be expected on Francois Lake.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 825 m3/s (gauge water level is 4.42 m) and are expected to continue a slow rise through the week.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water.  Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.