FLOOD WATCH: Nechako River

UPDATED: 21 May 2015 4:30 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

The BC River Forecast Centre is issuing a High Streamflow Advisory for:

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Nautley River near Fort Fraser (08JB003) is currently estimated to be flowing at 259 m3/s (gauge water level is 2.88 m) which is above the 50-year return period flow. The Nechako River at Vanderhoof (08JC001) is flowing at 558 m3/s (gauge water level is 4.85 m), which is above the 20-year return period flow. The Stellako River at Glenannan (08JB002) is flowing at 173 m3/s and gauge water level is 3.30 m.

Discharge from the Nechako Reservoir has been set to 345 m3/s this afternoon with Skins Lake Spillway continuing to be regulated to minimize the flood risk to Vanderhoof.

Continued snow melt in the range of 20 to 40 mm per day in the basin has resulted in steady rises in the Nautley, Nechako, Stellako, and Stuart rivers this week. Temperatures are expected to be seasonally warm through the weekend and there is potential for isolated thunderstorms due to continued convective activity in the Nechako region. Rain totals may be in the 5 to 10 mm range over the next two days and will diminish over the weekend with drier and warmer conditions expected next week.

Given the current conditions and the weather forecast for the weekend, flows on the Nautley River are expected to continue to increase through the weekend and is forecast to reach approximately 270-275 m3/s by Monday and potentially peak by next Wednesday. The Nechako River at Vanderhoof is expected to rise through the weekend and is forecast to reach 600 m3/s by Saturday and 610 m3/s by Monday, based on modelled flows at Nechako River below Cheslatta Falls of 330 m3/s (WSC 08JA017). The Stellako River at Glenannan is expected to see continued rises of 1 to 2 cm/day and similar rises may be expected on Francois Lake.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 845 m3/s (gauge water level is 4.48 m) and are expected to reach 900 m3/s by Saturday and 915 m3/s by Monday and continue to rise through next week. Water levels on the Fraser River at South Fort George (WSC 08KE018) are expected to increase through the weekend and reach a gauge elevation of 8.63 m by Monday.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water.  Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.