FLOOD WATCH: Nechako River

UPDATED: 22 May 2015 2:30 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

The BC River Forecast Centre is maintaining a High Streamflow Advisory for:

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Nautley River near Fort Fraser (08JB003) is currently estimated to be flowing at 260 m3/s (estimated gauge height of 2.89 m) which is above the 50-year return period flow. The Nechako River at Vanderhoof (08JC001) is flowing at 571 m3/s (gauge water level is 4.91 m), which is above the 20-year return period flow. The Stellako River at Glenannan (08JB002) is flowing at 171 m3/s and gauge water level is 3.29 m, and has been stable over the past day.

Discharge from the Nechako Reservoir has been set to 320 m3/s this morning with Skins Lake Spillway being managed to target a flow of approximately 600 m3/s at Vanderhoof.

Temperatures are expected to be seasonally warm through the weekend and there is potential for isolated thunderstorms due to continued convective activity in the Nechako region. Rain totals may be in the 5-15 mm range over the next 36 hours and will diminish towards the end weekend with drier and warmer conditions expected next week.

Given the current conditions and the weather forecast for the weekend, flows on the Nautley River may see modest increases through the weekend and are forecast to range from 260-270 m3/s (2.89-2.92 m gauge height). The Nechako River at Vanderhoof is expected to rise through the weekend and is forecast to reach 590 m3/s by Sunday (5.00 m gauge height) and 600 m3/s by early next week (5.05 m gauge height), based on modelled flows at Nechako River below Cheslatta Falls of 320 m3/s (WSC 08JA017). The Stellako River at Glenannan and Francois Lake are currently holding steady, and modest additional rises are possible over the weekend.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 857 m3/s (gauge water level is 4.51 m) and are expected to reach 900 m3/s (4.63 m gauge height) by early next week with continued rises through next week. Water levels on the Fraser River at South Fort George (WSC 08KE018) are expected to increase through the weekend and reach a gauge elevation of 8.00-8.4 m by Monday, below levels of concern.  There is the potential for rapid rises of the Fraser River at South Fort George (WSC 08KE018) through the week in response to forecasted hot temperatures and snow melt.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water.  Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.