FLOOD WATCH: Nechako River

UPDATED: 25 May 2015 3:00 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

The BC River Forecast Centre is maintaining the High Streamflow Advisory for:

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Nautley River near Fort Fraser (08JB003) is currently estimated to be flowing at 241 m3/s (estimated gauge height of 2.81 m) and has been steady over the weekend. The Nechako River at Vanderhoof (08JC001) is flowing at 590 m3/s (gauge water level is 5.0 m) and has been steady over the last 36 hours. The Stellako River at Glenannan (08JB002) is flowing at 171 m3/s and gauge water level is 3.29 m, and has been steady over the past 24 hours. The Stuart River near Fort St. James (08JE001) is currently at 300 m3/s (gauge water level is 2.65 m) and has been increasing approximately 6 m3/s (3.3 cm) per day over the past week.

The Water Survey of Canada (WSC) completed field measurements in the region late last week. According to the WSC, flows at the Vanderhoof (08JC001) gauge are accurate. However, in comparison to flow levels in 2007, the current rating curve at the station has shifted. This means that water levels are presently higher for the same flow rate in comparison to water levels in 2007. For example, in 2007, when the flow rate was 590 m3/s, the water level was approximately 4.8 m versus the currently observed 5.0 m. Note, the differences between the current rating curve and historical curve are non-linear and increase slightly at higher flow conditions.

Discharge from the Nechako Reservoir has been set to 340 m3/s this morning with Skins Lake Spillway being managed to target a flow near 600 m3/s at Vanderhoof at this time.

Within the Nechako region, rainfall totals recorded over the weekend were between 2 to 20 mm due to isolated thunderstorms. Early this week, continued instability and convective events will affect the region with less than 5 mm of total rain expected over the next 48 hours.

By Thursday, an upper low will sweep through the region bringing cooler temperatures and rain events with cumulative totals between 30 and 40 mm, from Thursday through Saturday. This estimate is conservative at this time and there is considerable uncertainty among the weather forecast models regarding the total rain expected later this week.

Given the current conditions and the weather forecast, flows on the Nautley River, Stellako River, and Francois Lake are expected to remain steady over the next three days. Flow levels may increase by the weekend depending on the amount of rain received later this week.

Pending any changes to the outflows of Skins Lake Spillway, the Nechako River at Vanderhoof is expected to remain steady this week and increase to 615 m3/s (increased water levels of 12 to 15 cm) by Friday depending on rainfall.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 898 m3/s (gauge water level is 4.63 m) and are expected to reach 930 m3/s (4.75 m gauge height) by Wednesday and continue to rise through the weekend.

Water levels on the Fraser River at South Fort George (WSC 08KE018) are at 8.397 m and are expected to increase to 8.8 m by Wednesday. The eastern headwaters of the Fraser River will see higher flows this week, slightly above the 2-year return period flow.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water. Livestock and other possessions of value should be moved to higher terrain if possible.

Due to the uncertainty in the precipitation forecasts for late this week, forecast flows may change substantially by Wednesday of this week. An updated advisory will be issued at that time. The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.