FLOOD WATCH: Nechako River

UPDATED: 27 May 2015 5:00 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

The BC River Forecast Centre is maintaining the High Streamflow Advisory for:

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Nautley River near Fort Fraser (08JB003) is currently estimated to be flowing at 240 m3/s (estimated gauge height of 2.81 m) and has been steady since Monday. The Stellako River at Glenannan (08JB002) is flowing at 168 m3/s and gauge water level is 3.28 m and has dropped slightly since Monday.

Discharge from the Nechako Reservoir was set to 500 m3/s yesterday with Skins Lake Spillway being managed to target a flow between 650 to 700 m3/s at Vanderhoof. The Nechako River at Vanderhoof (08JC001) is flowing at 588 m3/s (gauge water level is 5.0 m) and has been steady over the past two days.

The Stuart River near Fort St. James (08JE001) is currently at 310 m3/s (gauge water level is 2.71 m) and has been increasing approximately 6 m3/s (3.3 cm) per day over the past week.

The weather forecast indicates an upper low will move through the Nechako region by Thursday evening and may stall over the central Interior until Saturday. Rainfall totals expected over this period range from 10 to 20 mm over the Nechako basin and from 30 to 50 mm total rain near Burns Lake. Due to the instability of the weather system, there is potential for embedded thunderstorm cells which are capable of producing localized short-term heavy downpours of up to 20 mm per hour. At this point, weather models have uncertainty over the location of the upper low later in the week and therefore there is associated uncertainty of the location of heaviest rainfall during this event.

Given the current conditions and the weather forecast, flows on the Nautley River, Stellako River, and Francois Lake are expected to continue to slowly drop over the next three days. Water levels may increase 3 to 5 cm on the weekend depending on the amount of rain received in the headwaters of these water bodies.

Pending any changes to releases from Skins Lake Spillway and subject to total rainfall received, the Nechako River at Vanderhoof is expected to increase to 650 m3/s (gauge water level is 5.283 m) by Saturday.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 908 m3/s (gauge water level is 4.65 m) and are forecast to reach 930 m3/s (4.75 m gauge height) by Saturday. Flows are expected to reach 1050 m3/s by early next week due to increased flows from the upper basin and the Stuart River basin.

Water levels on the Fraser River at South Fort George (WSC 08KE018) are at 8.374 m and are expected to increase to 8.53 m by Saturday. The eastern headwaters of the Fraser River will see higher flows this weekend, slightly above the 2-year return period flow.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water. Livestock and other possessions of value should be moved to higher terrain if possible.

Due to the uncertainty in the precipitation totals forecasts for late this week, forecast flows may change substantially by Friday. An updated advisory will be issued at that time. The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.