FLOOD WATCH: Nechako River

UPDATED: 11 June 2015 3:30PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

Natural flows on tributaries to the Nechako River have dropped this week due to declining snowmelt runoff in the basin. Flows on the Stellako River at Glenannan (08JB002) are currently 119 m3/s (gauge water level of 3.0 m) and the gauge water level on the Nautley River near Fort Fraser (08JB003) is currently 2.74 m. Due to the backwater effect from the Nechako River, the estimated flows in the Nautley are between 160 to 170 m3/s.

Discharge from the Nechako Reservoir is currently at 500 m3/s. The Nechako River below Cheslatta Falls (08JA017) is currently at 484 m3/s. The Nechako River at Vanderhoof (08JC001) has dropped approximately 10 cm this week and is currently flowing at 654 m3/s (gauge water level is 5.30 m).

The weather forecast indicates the possibility of isolated showers over the weekend with daily totals up to 5 mm. Snowmelt runoff is expected to continue to decline as snow packs are close to fully melted for the season. Flows on the Nautley River, Stellako River, and at Francois Lake are expected to drop pending any large rain accumulations over the next week.

Flows on the Nechako River at Vanderhoof are expected to remain at or near the current conditions until discharge from the Nechako Reservoir decreases later this month.

The Stuart River near Fort St. James (08JE001) is currently at 361 m3/s (gauge water level is 2.97 m) and has leveled off.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 1028 m3/s (gauge water level is 4.96 m) and have decreased slightly this week. Flows at Isle Pierre are expected to remain around 1020 to 1030 m3/s into next week.

Water levels on the Fraser River at South Fort George (WSC 08KE018) are at 8.026 m and continue to decline.

Despite receding water levels in some locations, local residents are reminded to stay clear of streams until after water levels reach typical summer-time low flow conditions due to highly saturated and unstable riverbank conditions and swiftly flowing water.

The River Forecast Centre will continue to monitor conditions in the Nechako basin. Due to the low possibility of substantially increased flood risk expected over the next week, this advisory will remain in place until conditions warrant an update.