FLOOD WATCH: Nechako River

UPDATED: 01 June 2015 3:00 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Stellako River at Glenannan (08JB002) has dropped approximately 6 m3/s (2 cm), and is currently flowing at 157 m3/s (gauge water level at 3.22 m). Water levels at Francois Lake have also been reported to have eased this week. The Nautley River near Fort Fraser (08JB003) is currently estimated to be flowing at 244 m3/s (estimated gauge water level of 2.83 m). Flows appear to have increased slightly over the weekend, however given the drop in the Stellako River and rises in the Nechako River below Cheslatta Falls, rising levels may be the result of backwater at the Nechako River.

Discharge from the Nechako Reservoir is currently at 500 m3/s. The Nechako River at Vanderhoof (08JC001) has risen approximately 15 cm since Friday, and is flowing at 645 m3/s (gauge water level is 5.26 m).

The Stuart River near Fort St. James (08JE001) is currently at 338 m3/s (gauge water level is 2.85 m) and increased approximately 5 m3/s (2.5 cm) per day over the weekend.

Rainfall over the weekend was in the order of 10-20mm over the watershed. Rain has now eased into drier conditions, with limited organized rainfall expected through the week.

Given the current conditions and the weather forecast, flows on the Nautley River, Stellako River, and Francois Lake are expected to continue to slowly drop over the next five days as snowmelt input begins to recede. Further water level rises due to snowmelt alone are not probable. However water levels at these sites may increase if substantial rainfall occurs across the basin.

Based on outflows from the Skins Lake Spillway and continued inflows into the Nechako River below Cheslatta Falls, the Nechako River at Vanderhoof is expected to continue to rise through the early part of the week. Levels on the Nechako River at Vanderhoof are expected to increase an additional 10-15 cm to a water level of approximately 5.38 m (flow of approximately 670 m3/s) by Wednesday.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 982 m3/s (gauge water level is 4.85 m) and are expected to continue to rise due to increased flows from the upper basin. With increasing flows from the Stuart River and upstream on the Nechako, flows at the Isle Pierre gauge are forecast to reach approximately 1020 m3/s, or approaching a 20-year flow, by Thursday. Current modelling suggests that the Nechako River at Isle Pierre may peak later this week.

Water levels on the Fraser River at South Fort George (WSC 08KE018) are at 8.67 m and slowly declining. Flows are expected to remain in the 8.4-8.7 m range through the week.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water. Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.