FLOOD WATCH: Nechako River

UPDATED: 03 June 2015 3:00 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Stellako River at Glenannan (08JB002) has dropped approximately 22 m3/s (22 cm) since last week, and is currently flowing at 150 m3/s (gauge water level of 3.18 m). The gauge on the Nautley River near Fort Fraser (08JB003) is currently reading a water level of 2.86 m. Given the observed drops on the Stellako River and recent flow measurements taken at the Nautley River gauge, it is expected that the Nechako River is creating a backwater effect on the Nautley River near the gauge resulting in recorded flows that are erroneously high. Flows at the Nautley River gauge are estimated to be in the 200-220 m3/s range.

Discharge from the Nechako Reservoir is currently at 455 m3/s. The Nechako River below Cheslatta Falls (08JA017) has leveled off at a flow of 478 m3/s. The Nechako River at Vanderhoof (08JC001) has risen approximately 11 cm since Monday, and is currently flowing at 668 m3/s (gauge water level is 5.37 m).

Given the current conditions and the weather forecast, flows on the Nautley River, Stellako River, and Francois Lake are expected to continue to slowly drop through the rest of the week.

Flows on the Nechako River at Vanderhoof are expected to see additional rises of up to 5 cm today and tomorrow to reach the 670-680 m3/s range (gauge water level of 5.38-5.42 m).

The Stuart River near Fort St. James (08JE001) is currently at 348 m3/s (gauge water level is 2.90 m) and has continued to increase at approximately 5 m3/s (2.5 cm) per day. Steady rises are expected on the Stuart River into the weekend.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 1012 m3/s (gauge water level is 4.92 m) and rising at approximately 15 m3/s per day. The rate of rise in the Nechako at Isle Pierre and downstream is expected to slow into the weekend, as flows from the Vanderhoof and upstream level off. Flows at Isle Pierre are expected to reach 1030-1040 m3/s (20-year flow) over the weekend.

Water levels on the Fraser River at South Fort George (WSC 08KE018) are at 8.38 m and continue to decline. Flows on the Fraser River through Prince George are expected to remain below levels of concern through the rest of the week and weekend.

Local residents are reminded to stay clear of streams during the spring runoff period due to highly saturated and unstable riverbank conditions and swiftly flowing water. Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.