FLOOD WATCH: Nechako River

UPDATED: 05 June 2015 1:00 PM

The BC River Forecast Centre is maintaining a Flood Watch for:

- Nautley River near Fort Fraser

- Nechako River at Vanderhoof

- Nechako River at Isle Pierre and downstream to the confluence with the Fraser River

The Stellako River at Glenannan (08JB002) continues to decrease and is currently flowing at 142 m3/s (gauge water level of 3.13 m). The gauge water level on the Nautley River near Fort Fraser (08JB003) is currently 2.86 m. Due to the backwater effect from the Nechako River, the estimated flows in the Nautley are in the 200 to 220 m3/s range.

Discharge from the Nechako Reservoir is currently at 455 m3/s. The Nechako River below Cheslatta Falls (08JA017) has leveled off at a flow of 480 m3/s. The Nechako River at Vanderhoof (08JC001) has leveled off over the past 24 hours and is currently flowing at 676 m3/s (gauge water level is 5.40 m).

The weather forecast indicates the possibility of isolated showers today with totals up to 5 mm. The following 5 days are forecast to be warm and dry and snowmelt from this winter’s snow pack is nearly finished for the region. Flows on the Nautley River, Stellako River, and Francois Lake are expected to slowly drop through next week.

Flows on the Nechako River at Vanderhoof are expected to remain at or near the current conditions through to mid-next week.

The Stuart River near Fort St. James (08JE001) is currently at 357 m3/s (gauge water level is 2.95 m) and has continued to increase at approximately 5 m3/s (2.5 cm) per day. The rate of rise is expected to slow down through the weekend but water levels will not peak until later next week, possibly 10 to 15 cm higher than current conditions.

Flows on the Nechako River at Isle Pierre (WSC 08JC002) are at 1036 m3/s (gauge water level is 4.98 m) and have increased 10 m3/s over the last 24 hours. The rate of rise is expected to slow over the weekend at the Isle Pierre gauge. Flows at Isle Pierre are expected to reach between 1040 and 1050 m3/s (increase 2 to 4 cm) over the weekend.

Water levels on the Fraser River at South Fort George (WSC 08KE018) are at 8.26 m and continue to decline. Flows on the Fraser River through Prince George are expected to remain below levels of concern through next week.

Despite receding water levels in some locations, local residents are reminded to stay clear of streams until after water levels reach typical summer-time low flow conditions due to highly saturated and unstable riverbank conditions and swiftly flowing water. Livestock and other possessions of value should be moved to higher terrain if possible.

The River Forecast Centre will continue to monitor conditions and will provide updates as conditions warrant.