TEMPLATE

Application Requirements

for a

Mines Act Permit

and an

Environmental Management Act Permit

for proposed mining projects

| Version | Date | Comments |
| --- | --- | --- |
| 0.1 | September 14, 2010 | First Draft – working copy |
| 0.2 | September 15, 2010 | Second Draft – for review |
| 0.3 | September 16, 2010 | ESD comments |
| 0.4 | September 16, 2010 | AB comments |
| 0.5 | September 27, 2010 | JCF comments |
| 0.6 | October 5, 2010 | Third Draft – for second review |
| 0.7 | October 12, 2010 | Groundwater comments (MW). |
| 0.8 | November 1, 2010 | For distribution to EAO re: Kitsault Synchronous Permitting |
| 1.0 | July 4, 2011 | Remove word “Draft” from TOR with understanding that TOR will be continually updated. |
| 1.1 | March 30, 2012 | Revisions for inclusion in Guide to Coordinated Authorizations for Major Mines |

Application Requirements

for a

Mines Act Permit

and an

Environmental Management Act Permit

for proposed mining project: ____________________

Prepared By:

XXXXXX Company Ltd.

Submitted to:

Chief Inspector of Mines

British Columbia Ministry of Energy, Mines and Petroleum Resources (MEMPR)

Victoria, BC

&

Director, Environmental Management Act

British Columbia Ministry of Environment (MOE)

&

______ Mine Development Review Committee

_________ Region

_________, BC

XXXX, XX, 2010

Preface

This Application Requirements document sets out the information that will be required to support a Mines Act (MA) permit and a waste discharge permit under the Environmental Management Act (EMA) for a proposed mining project. The combined requirements are intended to reduce overlap in application requirements and enable bundled applications to be submitted together for review under the coordinated authorizations process.

Instructions to Proponent:

This Application Requirements template is designed for MA/EMA authorizations to enable the construction, operation and closure of a major mine. The template should be modified specifically for each project after discussion with and advice from the project’s mine development review committee.

The EMA permit application can seek authorization for a major mine’s construction stage (sediment control and blasting residues) and the operations stage (metal/chemical residues and contact water and tailings). Alternatively, the proponent may take a two step approach to the EMA permit by seeking authorization for only the construction stage and deferring authorization for the operations stage until engineering is sufficiently detailed to support an application for amendment to the permit. See the preamble to sections 7, 8 & 9 for guidance on this two-step approach.

In addition to this document, ensure that an Application form for an Authorization to Discharge Waste under the Environmental Management Act is completed and application fees are included. The form may be obtained at:  http://www.env.gov.bc.ca/epd/waste_discharge_auth/app_forms/pdf/approval-application.pdf

Note that when conducting any emission dispersion modeling it is important that models used are those accepted by the province. Please verify acceptability of models with the regional Environmental Protection staff to prevent ineffective use of your time and financial resources. The pre-application meeting and follow-up terms of reference discussions are to review such issues.  The ministry’s Municipal Sewage Regulations also provide guidance on how to deal with terms of reference for increasingly complex projects, using sewage and aquatic systems as examples. The Municipal Sewage Regulation Environmental Impact Study Guideline document may be obtained at: http://www.env.gov.bc.ca/epd/epdpa/mpp/pdfs/EIS_Guideline_Dec2000.pdf

Table of Contents

EXECUTIVE SUMMARY	7

1. INTRODUCTION	8

1.1 Proponent	8

1.2 General Application Background	8

1.3 Project Overview	8

1.3.1 Project History	8

1.3.2 Overview of Products and Markets	8

1.3.3 Location and Access	8

1.3.4 Mine Components and Off-Site Infrastructure	8

1.3.5 Mine Development and Operations	8

1.3.6 Regulatory Framework	9

1.3.7 Mine Design and Assessment Team	9

1.3.8 Maps, Figures, Etc.	9

1.3.9 EA issues and commitments	9

1.3.10 Wells	9

2. BASELINE INFORMATION	10

2.1 Climate	10

2.2 Geology	10

2.2.1 Regional Geology	10

2.2.2 Deposit (Ore) Geology	11

2.3 Topography, Surface Drainage Features, and Natural Hazards	11

2.4 Surface Water and Groundwater Hydrology	11

2.4.1 Purpose of all this work	11

2.4.2 Identify and describe connection to surface water and identify sw and gw dependent features (quanity and quality). Estimate of relevant fluxes.	12

2.5 Water Quality (surface water and groundwater)	12

2.6 Fisheries and Aquatic Resources	13

2.6.1 Baseline Information – Aquatic Resources	13

2.6.2 Baseline Information – Fisheries	13

2.7 Surficial Geology, Terrain and Soils	13

2.7.1 Surficial Geology and Terrain Mapping	13

2.7.2 Soil Survey and Soil Characterization for Reclamation	14

2.7.3 Soil survey information provided will include:	14

2.7.4 Baseline Soil Metal Concentrations	14

2.8 Vegetation and Wildlife	14

2.9 Land Status and Use	14

2.10 Land Capability	14

2.11 Archaeology and Cultural Use	14

3. MINE PLAN	16

3.1 Mine Plan Overview	16

3.2 Development Sequence and Schedule	16

3.3 Existing Development	16

3.4 Water Management Plan	16

3.4.1 Diversion Structures and Control Ponds	16

3.5 Mine Facility Designs and Development	17

3.5.1 Open Pit	17

3.5.2 Processing Plant (Mill) and Associated Facilities	18

3.5.3 Tailings Management Facility and Associated Infrastructure	18

3.5.4 Waste Rock Storage Facility	18

3.5.5 Low Grade Ore Stockpike	19

3.5.6 Soil Stockpiles	19

3.5.7 Maintenance shop, Fuel Stations and Associated Support Facilities	19

3.5.8 Roads (new and upgrades)	19

3.5.8.1 Proposed Mine Access	19

3.5.9 Power Supply and Distribution	19

3.5.10 Explosives	20

3.5.11 Additional Ancillary Facilities	20

3.5.12 Fish Habitat Compensation Works	20

3.6 Traffic Control Plan	20

3.7 Detailed Five Year Mine Plan	20

3.8 Conceptual Life of Mine Plan	20

4.  ENVIRONMENTAL MANAGEMENT PLAN	21

4.1 Environmental Monitoring and Management Plan	21

4.2 Construction Management Plan	21

4.3 Surface Erosion Prevention and Sediment Control Plan	21

4.4 Fuel Management Plan	23

4.5 Waste Management Plan	23

4.6 Chemicals and Materials Storage and Handling Plan	26

4.7 Wildlife Management Plan	26

4.8 Health and Safety	27

4.9 Traffic Management Plan	27

4.10 Emergency Response Plan	27

4.11 Archaeological Impact Mitigation and Management Plan	27

5. RECLAMATION PROGRAM	28

5.1 End Land Use Objectives	28

5.2 Soil Handling Plan	28

5.3 Environmental Monitoring and Surveillance	29

5.4 Conceptual Final Reclamation Plan	29

5.5 End Land Use Objectives	29

5.6 Productivity or Capability Objectives	29

5.7 Long-term Stability	29

5.8 Treatment of Structures and Equipment	29

5.9 Waste Dump Reclamation	29

5.10 Tailings Reclamation	30

5.11 Pit Reclamation	30

5.12 Watercourse Reclamation	30

5.13 Road Reclamation	30

5.14 Trace Element in Soils and Uptake in Vegetation	30

5.15 Disposal of Toxic Chemicals	30

5.16 Contaminated Site Requirements	30

5.17 Temporary Closure Care and Maintenance and Monitoring	31

5.18 Operational and Post Closure Monitoring	31

5.19 Detailed Five Year Mine Reclamation Plan	31

5.20 Conceptual Life of Mine Reclamation Plan	31

5.20.1 Information sources used to develop revegetation plan will include:	31

5.20.2 Closing of Wells	31

6. METAL LEACHING / ACID ROCK DRAINAGE (ML/ARD) ASSESSMENT	32

6.1 ML/ARD Prediction and Prevention Plan	32

6.2 Materials Handling Plan	33

6.3 Mine Waste Sequencing and Waste Placement Plan	33

6.4 Mitigation Plans	33

6.5 Water Quantity and Quality Monitoring Plan	34

Preamble to Sections 7, 8 & 9 for Two-Step EMA Permitting	35

7. DISCHARGES AND TREATMENT	36

8. ENVIRONMENTAL EFFECTS ASSESSMENT FOR EMA	36

8.1 General	36

8.2 Areas of Specific Topics	37

8.2.1 Environmental Effects Assessment	37

8.2.2 Fisheries and Aquatic Resources	37

8.2.3 Impacts to humans	37

8.2.4 Surface Water and Groundwater Hydrology	37

8.2.5 Water Quality Modeling	38

9. PROPOSED DISCHARGE AND ENVIRONMENTAL EFFECTS MONITORING FOR EMA	39

10. RECLAMATION COST ESTIMATES	40

10.1 Post Closure Monitoring	40

10.2 Post Closure Maintenance (excluding water treatment)	40

10.3 Post-Closure Maintenance (Water Treatment)	40

11. OTHER COMMENTS	41

11.1 Referencing to Earlier Submitted Information	41

11.2 References	41

# EXECUTIVE SUMMARY

Provide an executive summary that briefly describes the proposed project, identifies the authorizations being applied for and describes how the application meets the application requirements document developed with advice from the mine development review committee.

# 1. INTRODUCTION

Précis

This section will provide contextual background information on the mining project, including Proponent identification, application background, mine overview and development proposal, regulatory framework, and the mine design and assessment team.

## 1.1 Proponent

- Proponent information (i.e., name, address, phone, fax, email)

- Name of the company representative managing the Project

- Company incorporation and structure of the company

- Overview of company:

  - Description of the operating and parent corporation proposing to develop and operate

  - Registered legal name and registered address of company

  - Head office contact name and phone and fax numbers

  - Site office location and contact name, phone, fax numbers and email address

- Corporate Environmental Policy

## 1.2 General Application Background

- General introduction to the  Application and its structure

- Summary of the regulatory environmental assessment review completed to date and approvals received, and

- Summary of key commitments applying to the project development.

## 1.3 Project Overview

### 1.3.1 Project History

Describe project history leading up to the application, including a list of previous related reports.

### 1.3.2 Overview of Products and Markets

### 1.3.3 Location and Access

### 1.3.4 Mine Components and Off-Site Infrastructure

Introductory descriptions of the key mine components defined as part of the mine, and off-site infrastructure to be permitted under other legislation.

Concentrate haulage routes will be described along with associated detailed map.

### 1.3.5 Mine Development and Operations

Briefly outline the proposed mine development and operations including proposed discharges and locations.

### 1.3.6 Regulatory Framework

- A summary of key federal and provincial government legislation applicable to the mine and agencies responsible for implementation

- Outline of required licenses/permits/authorizations needed for development and/or operation

### 1.3.7 Mine Design and Assessment Team

This will identify the consultants and individuals comprising the design and assessment team, and their responsibilities and application contributions.

### 1.3.8 Maps, Figures, Etc.

Include updated maps (plan and section), diagrams, photographs, videos and graphs, as appropriate, outlining current conditions. Provide many supporting maps of an appropriate scale for interpretation. Supporting maps must provide sufficient design information to assess the adequacy of proposed works, proposed monitoring and contingency measures.

### 1.3.9 EA issues and commitments

Identify any relevant issues and commitments from the EA process.

### 1.3.10 Wells

It is the policy of MEMPR and MOE that any wells drilled (e.g., geotechnical boreholes, monitoring wells, remediation wells, dewatering wells, water supply wells be constructed, maintained and closed in accordance with the requirements under the Water Act and minimum requirements in the Ground Water Protection Regulation.

As a condition of the permit, all monitoring, water supply and dewatering wells should be entered into WELLS by the applicant (via eWELLS). All water quality should be uploaded into EMS by the applicant.

# 2. BASELINE INFORMATION

Précis

This section will provide a summary of the existing baseline receiving environment quality, and review any existing monitoring-and-impact assessments relevant to the proposed discharges. The summary should review the location and physical, chemical and biological characteristics of the receiving environment, focusing on sensitive receptors that are relevant to the potential project impacts.

The applicant should also identify receiving environment information gaps that will be relevant to the assessment, by considering issues such as what air or water quality data will be needed to set environmental quality objectives or determine the cumulative effects of the operation. Significant gaps may require the collection of receiving environment data, stressing the need for early discussion with Provincial Government staff.

Notes:

- Clearly summarize findings of baseline information and collate with work done post-EA.

- All raw data must be included in the appendices and electronically.

- If the mine plan has changed, provide a summary of changes with regard to the adequacy of the environmental data previously collected.

Identify issues related to those topics that have relevance to the permit process in the sections below when presenting information on climate, geology, topography and surface and groundwater hydrology.

## 2.1 Climate

- Summary of average precipitation and temperature conditions with a description of techniques used to determine them, updated from information presented during EA with data collected subsequent to the EA data cut-off.  Supporting data to be included as a CD data appendix.

- Predicted ranges of climatic parameters relevant to mine facility design and operations (i.e. focusing on hydrological parameters).

## 2.2 Geology

### 2.2.1 Regional Geology

- Regional geological setting

- An overview of the geology of the area, with emphasis on the regional framework. This will include a description of the tectonic belt(s), terrain(s), physiography, regional metamorphism and structure.

### 2.2.2 Deposit (Ore) Geology

- Summary of the mine site geology, including descriptions of major rock units, stratigraphy, structure, metamorphism, geochemistry, paleontology and details about the ore deposit.

- Detailed stratigraphic description.

- Ore deposit information will include:

  - Comments on the ore mineralogy including alteration type, deposit character, deposit classification and age of mineralization

  - General ore controls, and

  - Average assay values and reserve information (proven, probable and possible).

- Summary of geochemistry (details to be provided in ML/ARD section).

## 2.3 Topography, Surface Drainage Features, and Natural Hazards

- Description of pre-mine topography and surface drainage features.

- Maps at a suitable scale depicting drainage divides, areas of groundwater discharge, wetlands and notable topographic features.

- Accompanying descriptions will show the range of pre-mine slope configurations and typical slope cross-sections.

- Additional regional mapping will also be provided, showing the entire drainage basin(s) in which the mine will be located.

- Information regarding any natural hazards such as snow avalanches, landslides and earthquakes will be provided as specific to the mine.

## 2.4 Surface Water and Groundwater Hydrology

- A detailed summary of the existing surface hydrological and groundwater conditions is required.

- A map showing the hydrology stations as well as the groundwater monitoring locations with an accompanying study design showing rationale for site selection and monitoring parameters is required.

- Provide predicted ranges of hydrological parameters relevant to mine facility design and operations.

- Provide a map of groundwater elevations and flowpaths for seasonal variations (at a minimum spring freshet and winter low flows).

- Provide a discussion of potential effects to quantity and quality of surface flows to groundwater.

### 2.4.1 Purpose of all this work

For groundwater, the applicant should characterize the resource sufficiently to address all the relevant issues related to groundwater in the mine and discharge permitting processes. Some fundamental items include:

- The conceptual hydrogeologic model of the mine area. What are the assumptions in the conceptual hydrogeologic model and how are they justified?

- What geological units does the groundwater occur in? What are the units’ characteristics? For example, does the groundwater occur in a fractured bedrock? If in a fractured bedrock environment, what is the evidence that the groundwater can be assessed using a porous medium approach versus a discrete fractured approach in the conceptual or numerical model. What is the implication of a fractured porous medium on anisotropy in characteristics of the geological units?

- What are the boundary conditions?

### 2.4.2 Identify and describe connection to surface water and identify sw and gw dependent features (quanity and quality). Estimate of relevant fluxes.

- Include maps and relevant sections showing groundwater elevations and inferred directions of groundwater flow. Address the implications of seasonal conditions on groundwater flow.

- What are the uncertainties and how will they be addressed?

- Hydrogeological interpretation must be done by a Professional Engineer or Professional Geoscientist registered in BC and with competency in hydrogeology. For characterizing surface water and groundwater hydrology, the applicant must also include information on standard procedures for data collection and accuracy of the data.

## 2.5 Water Quality (surface water and groundwater)

Provide a summary of pre-mine surface and groundwater quality, with references to further details provided in the EA Application.

The MA/EMA applications will require a detailed summary of water quality.  This section will need to detail existing water quality at all individual sampling locations. Present results and discuss their validity.  A comparison of existing water quality conditions and modeled water quality predictions to provincial water quality guidelines is required.  If guidelines are naturally exceeded during baseline conditions, then site specific water quality objectives need to be established using approved methods.  For information on site specific water quality objectives, refer to the following documents:

- Principles for Preparing Water Quality Objectives in British Columbia.  http://www.env.gov.bc.ca/wat/wq/BCguidelines/principles.html

- Developing Water Quality Objectives in British Columbia: A User's Guide.  http://www.env.gov.bc.ca/wat/wq/BCguidelines/wq_ob_user_guide/usersguide.html

- Methods for Deriving Site-Specific Water Quality Objectives in British Columbia and Yukon.  http://www.env.gov.bc.ca/wat/wq/BCguidelines/effects_ratio/effectsratio.html

A detailed map of water quality sampling locations, discharge points, and rationale for site selection is required. Raw data will need to be compiled and placed in an appendix. Include a study design table listing sample site locations and sample dates.

## 2.6 Fisheries and Aquatic Resources

This section will provide information about the receiving environment prior to discharges. To be useful in identifying potential impacts from the mine, the Environmental Effects Monitoring (EEM) Program must adequately characterize time zero in an ongoing impact assessment.  Appropriate biological monitoring tools must be used, and sufficient data must be collected and presented to demonstrate that the EEM program will be able to detect significant change. If significant change is detected, decisions can then be made to take appropriate management actions. Raw data will need to be compiled and placed in an appendix.

### 2.6.1 Baseline Information – Aquatic Resources

- Description and map of aquatic resources, including access roads and utility corridors.

- Summary discussion of aquatic resources related data collection for the EA Application.

- Summary of any post-EA data collected related to aquatic resources.

- Provide an inventory of potential biological receptors.

- All raw data must be included in the appendices to allow calculation of various metrics in the ongoing assessment.

### 2.6.2 Baseline Information – Fisheries

- Description and map of fisheries resources, including access roads and utility corridors.

- Summary discussion of fisheries related data collection for the EA Application (with references to detailed information in the EA Application).

- Summary of any post-EA data collected related to fisheries and the fish habitat compensation plan.

- Outline fish habitats (spawning, overwintering, rearing, etc.) downstream of any discharges that may be exposed to some degree of mine-related effluent.

- All raw data must be included in the appendices to allow calculation of various metrics in the ongoing assessment.

- Conceptual Fish Habitat Compensation Plan, if required. A brief description of the approved conceptual Fish Habitat Compensation Plan, if applicable.

- Rationale for fish habitat compensation recommendations and alternative plans.

## 2.7 Surficial Geology, Terrain and Soils

- Surficial Geology and Terrain Mapping,

- Soil Survey and Soil Classification for Reclamation, and

- Baseline soil metal concentrations.

### 2.7.1 Surficial Geology and Terrain Mapping

This subsection will provide a brief summary of surficial geology and terrain mapping completed at a scale of 1:5,000 or as appropriate using the ‘Terrain Classification System for British Columbia’.

### 2.7.2 Soil Survey and Soil Characterization for Reclamation

This subsection will provide a brief summary of soil survey for the mine ‘footprint’. The summary will include supportive technical data including soil classification and soil profile descriptions.

### 2.7.3 Soil survey information provided will include:

- Identification and mapping of soil units;

- Characterization of topsoil and subsoil for suitability as a growth media for reclamation;

- Location, depth and volumes of soil types; and

- Potential soil and subsoil salvage locations

- Discussion of potential for erosion capabilities.

### 2.7.4 Baseline Soil Metal Concentrations

## 2.8 Vegetation and Wildlife

This subsection will provide a brief summary of the Terrestrial Ecosystem Mapping (TEM), Predictive Ecosystem Mapping (PEM), and the location of rare plants and ecosystems.   Wildlife information will include wildlife habitat suitability mapping, results of aerial wildlife surveys, and a description and location of any key wildlife features or habitats. Bioterrain Mapping and Vegetation Metals analyses should also be included here. Provide a description of onsite and adjacent terrestrial and aquatic ecosystems, and an inventory of potential biological receptors, including human populations. Show evidence of consultation with First Nations with particular regard to wildlife and vegetation.

## 2.9 Land Status and Use

Mapping and a summary will be provided of pre-mine land surface and mineral rights, and licensed or permitted users such as forestry, guides, outfitters, and trappers. Existing transportation routes (e.g. roads or waterways) within or adjacent to the minesite will be shown and described.  Any known First Nation or informal users who are not necessarily licensed (e.g. recreational users) will also be described. Include a description of any known local land use and settlement patterns. Provide a description of onsite and adjacent terrestrial and aquatic ecosystems, and an inventory of potential biological receptors, including human populations.

## 2.10 Land Capability

Mapping and descriptions will be provided of existing land capabilities which will be used to describe potential end land use(s) following reclamation.

## 2.11 Archaeology and Cultural Use

Maps and descriptions should be provided for all identified archaeological sites in the project impact zone.  Maps based on those provided in the final AIA report may be adequate as overview maps. Detailed maps at a 1:500 scale should be provided for any sites that are to be subjected to additional systematic data collection under HCA Section 14 permits.

The site descriptions can be provided in table form, and should include Borden number, general location, previous work [whether an AIA was completed, site collected completely, or site avoided], and proposed mitigation [no further work, alteration under S12, or mitigation under S14].

A description of the required HCA permitting and concurrent archaeological activity should be provided, including (where applicable) the main terms and conditions of HCA Section 14 Investigation Permit(s) and the methodologies for HCA Section 14 Inspection Permit(s), the proposed HCA Section 12 Alteration Permit(s), and the Section 14 Inspection Permit for incidental finds during construction.

A description of (and a commitment to) a chance-finds procedure and education of the construction crew should be provided.  Terms of Section 12 site alteration permits and Section 14 inspection permits for incidental finds during construction, state that no land alteration must occur without an archaeologist on site as monitor, and the archaeologist on site will have the authority to stop excavation as deemed necessary so that any archaeological resources can be handled under the terms of the permit.

# 3. MINE PLAN

Précis

This section will present details of the proposed mine plan.

## 3.1 Mine Plan Overview

A brief overview of the mine plan will be provided, along with supporting mapping.

## 3.2 Development Sequence and Schedule

Proposed mine development sequence and schedule

## 3.3 Existing Development

Describe clearing and infrastructure currently in place within the mine area as a result of previous and ongoing mineral exploration activities.

## 3.4 Water Management Plan

Précis

Geotechnical and hydraulic stability assessments will be provided for all water diversion, interceptor, and sediment retention structures. Proposed monitoring and maintenance programs will also be described.

### 3.4.1 Diversion Structures and Control Ponds

3.4.1.1 Overview

3.4.1.2 Sediment Control Pond Designs

3.4.1.3 Diversion Structure Designs

3.4.1.4 Use of Existing Drainages

3.4.1.5 Structure Maintenance

3.4.1.6 Site Investigations and Additional Studies

3.4.1.7 Mine Water Source and Use

Include the delineation of the watershed or source area boundary upstream of the proposed water intake (if surface water, including a spring) or capture zone (if on a well). Methods for delineating the source area for a water supply spring or a well capture zone is presented in:

- Defining the source area for a water supply spring (http://a100.gov.bc.ca/pub/acat/public/viewReport.do?reportId=17862)

- Step 2 of the Well Protection Toolkit (http://www.env.gov.bc.ca/wsd/plan_protect_sustain/groundwater/wells/well_protection/wellprotect.html).

- If the source is a water supply well, provide the details of the well’s location and construction, yield and water quality testing and the source of the groundwater in relation to the geological units.

- Identify and discuss any potential impacts of the proposed mining and waste discharge activities on the mine water source and use.  Identify any relevant conditions in the permits from the regional health authority for the wells construction and the water system’s operation.

3.4.1.8 Water Balance

Identify and discuss potential impacts of mining and waste discharge on the water balance at the mine (including impacts on surface water, groundwater and water-dependent features), how this was determined and uncertainties related to this assessment and how uncertainties would be addressed.

3.4.1.9 Construction Overview

## 3.5 Mine Facility Designs and Development

Précis

This section will contain facility designs and construction specifications for all major mine structures, including tailings dams, water retaining dams, waste rock dumps, overburden, soil stockpiles and containment ditches, in accordance with Mine Code requirements.  Locations and configurations of these facilities will be shown, and construction materials and methods will be described.  Supporting data from the geotechnical field investigations will be provided in an appendix.

Note: planning needs to be far enough along in order to properly assess impacts to the environment from the associated discharges or wastes generated from each component.

Final design reports, containing additional design details not fundamental to the environmental review - such as construction materials and specifications, construction methods and sequencing, additional supporting information from site investigations and laboratory testing, design parameters, engineering design criteria, stability analysis and source of construction materials - will be submitted to the Chief Inspector when complete for final engineering review and approval prior to construction.  Copies will also be made available to NWMDRC members on request.

All dams will be designed for safety with reference to the 2007 Canadian Dam Association, Dam Safety Guidelines

### 3.5.1 Open Pit

- Pit limits, pit slope geometry and pit slope design criteria

- Supporting information obtained from geotechnical and other site investigations and laboratory testing, design methodology, stability analysis, sensitivity analysis and design criteria will be submitted in a separate report for geotechnical review

- Preliminary pit slope designs, and

- Pit slopes for phased pit expansions using the detailed geology and geotechnical information obtained from exposed pit slopes, additional geotechnical boreholes and experience with the interim walls

- Description of equipment to be used

- Information on number, location, spacing, design of dewatering wells, information on their proposed construction, operation and closure, volumes to be pumped, predicted area of drawdown, expected water quality of the pumped water, where water will be discharged to and expected impacts and how to address them. Details about how to monitor conditions outside the pit during and after mining.

### 3.5.2 Processing Plant (Mill) and Associated Facilities

- Process description. Describe inputs, products and non-product outputs for all stages of operations. Non-product outputs may include materials sent to offsite facilities, or lost to the environment.

- Process Design Criteria

- Flow sheets showing process streams, quantities and significant equipment

- Process reagents

- Identification and descriptions of hazardous products, with reference to relevant sections of the Environmental Management Plan that address their safe handling and storage.

- Foundation Design Criteria and philosophy

- Facility location drawing and description

- Supporting information obtained from site investigations and laboratory testing, design parameters, design criteria, foundation requirements and a summary of construction specifications will be submitted in a separate report for geotechnical review.

### 3.5.3 Tailings Management Facility and Associated Infrastructure

- Tailings Management Facility plans and sections at appropriate scales, showing the proposed tailings impoundment facility and dam profiles projected over the life of the mine.

- Descriptions of main construction materials and methods, and characteristics of the surface materials.

- Results of the geotechnical site investigation program, confirming primary design features and foundation conditions at the dam locations.

- Identification of any findings that are significantly different from what was predicted during EA – particularly as it relates to seepage.  If there are significant differences, identification of whether or not design modification(s) or other form of mitigation is proposed to address this.

- Supporting data from the geotechnical field investigations and associated laboratory work will be provided in an appendix.

- Address potential for groundwater contamination from this activity and present and discuss plan to monitor and mitigate GW contamination.

### 3.5.4 Waste Rock Storage Facility

- Plans and sections detailing proposed waste rock dumps, projected over the life of mine.

- Final post-mine waste rock dump configurations following resloping.

- Descriptions of how waste rock dumps will be designed – including any proposed cover and/or drainage collection system(s) – to address potential ML/ARD – with cross references to the relevant ML/ARD subsections that address this in greater detail.

- Description of operating practices, including any proposed special handling, with cross-references to relevant ML/ARD subsections if applicable.

- All waste dumps and soil stockpiles will be designed for safety with reference to the Interim Guidelines of the British Columbia Mine Waste Rock Pile Research Committee.

- Address potential for groundwater contamination from this activity and present and discuss plan to monitor and mitigate GW contamination.

### 3.5.5 Low Grade Ore Stockpike

A separate dump may be required to store low grade ore for possible future processing.

- Address potential for groundwater contamination from this activity and present and discuss plan to monitor and mitigate GW contamination.

### 3.5.6 Soil Stockpiles

Soil storage locations, configurations, and anticipated volumes will be identified in the Application.

### 3.5.7 Maintenance shop, Fuel Stations and Associated Support Facilities

### 3.5.8 Roads (new and upgrades)

### 3.5.8.1 Proposed Mine Access

#### 3.5.8.2 Mine Haul Roads

This will include:

- General description of mine site haulage roads

- Methods to be followed in the construction of mine site haulage roads

- Methods to be followed in the selection of construction materials for haulage roads to prevent the use of potentially ARD generating (PAG) rock, and cross-referencing ML/ARD subsections that describe identification and operational separation methods

### 3.5.9 Power Supply and Distribution

#### 3.5.9.1 Proposed powerline to mine site

#### 3.5.9.2 Onsite powerline infrastructures including substation and power distribution

This will include:

- Description and mapping of utility corridors

- Description and mapping of the onsite substation

- Description of onsite power distribution

### 3.5.10 Explosives

A description of onsite operational explosives storage and/or manufacturing facility will be provided, along with a description of any plans to retain a licensed explosives contractor.  This section will also include a description of any plans for explosives use during mine construction.

An evaluation is required of explosives residuals in the discharge(s) from the site.

### 3.5.11 Additional Ancillary Facilities

All additional minesite structures, including on-site accommodation and offices, will be described in terms of location and construction. Items of particular relevance to the reclamation plan regarding locations, foundations, and nature of construction (e.g. movable modular units or ‘permanent’ structures) will be described.

### 3.5.12 Fish Habitat Compensation Works

General description of all works related to approved Fish Habitat Compensation Plan

## 3.6 Traffic Control Plan

- General description of the mine site traffic control plan including:

  - radio frequencies

  - maximum allowable speeds for the vehicles in use

  - rules for passing

  - "stop" and "yield" locations

  - priority rules for various vehicles

  - rules for night operation

  - maximum operating grades

  - emergency run-off protection

  - shoulder barriers, and

  - any other information required by the District Inspector.

- Traffic control plan for any access road(s) to the mine site.  This section will describe access restrictions, load restrictions (if any), radio frequencies to be used by local traffic.  Route maps will be provided showing key locations or turns.

## 3.7 Detailed Five Year Mine Plan

This subsection will describe detailed year by year five year mine plan.

## 3.8 Conceptual Life of Mine Plan

This subsection will briefly describe conceptual Life of Mine (LOM) mine plan.

# 4.  ENVIRONMENTAL MANAGEMENT PLAN

Précis

This section will provide a brief summary of an overall Environmental Management Plan (EMP) applicable during all phases of the project from pre-construction to construction, operation, closure and post closure / reclamation.  The detailed Environmental Management Plan will be a living document and will be updated as appropriate during mine life.  The detailed EMP will be appended to the MAPA to serve as a stand-alone document.

## 4.1 Environmental Monitoring and Management Plan

- Environmental Policy Statement

- Environmental Management Roles and Responsibilities

- Statutory Requirements – applicable local, provincial or federal environmental standards and guidelines including permit requirements, regulations, and orders

- Environmental Standards and Procedures – all applicable sector-specific standards, guidelines, best management practives, and codes of practice (e.g., Responsible Care, CSA, ASTM, RISC, GWPR).

- Organizational Structure

- Training

## 4.2 Construction Management Plan

- Project Schedule

- Construction Coordination and Management

- Construction Workforce

- Proposed phases of the project including site preparation and construction (e.g. sediment control, open burning, waste rock handling, soil salvaging, interim domestic effluent and solid waste treatment during construction, etc.),

## 4.3 Surface Erosion Prevention and Sediment Control Plan

This will include the following specific areas:

- Mine (plant site, pit, waste dumps, tailings impoundment) Refer to information box: Effluent Authorizations During Construction

- Access Road

- Utility Corridors

The plan will provide a description of how erosion will be prevented / minimized during construction and throughout the mine life, and methods to be used to contain detached sediment within the minesite.

Provide erosion control/sediment control plans for disturbed surfaces and soil stockpiles. A response plan will be prepared to respond with knowledge and materials (seed, fertilizer, silt fences, settling ponds and, if appropriate, flocculant), and will be included in the Application.

Potential for sediment release from internal mine sloped structures such as waste rock dumps and any segments of pit walls will be addressed primarily through the water management plan (refer to Section 3.4).

Effluent Authorizations During Construction

For effluent authorizations during construction, applications under the EMA must include completion of the application form, a $200 processing fee and supporting documentation as required by regional office of Environmental Protection.  Notice of the application must also be given in accordance with the requirements of Public Notification Regulation. Additional information on applying for an authorization under EMA can be found at the following website: http://www.env.gov.bc.ca/epd/waste_discharge_auth/intro.htm

Applicants are strongly encouraged to contact the regional office for Environmental Protection for specific guidance.  In general, the following information should be provided:

Details regarding the location and sizing of any proposed sediment control ponds are required.  The additional information should include:

- preliminary design drawings and details on spillway construction for a typical pond,

- details on pond sizing including catchment area size for each pond and expected flows,

- design settling and particle size analysis from the catchment area of the ponds,

- retention time and expected discharge water quality, and

- sampling, inspection and maintenance that will be undertaken.

Provide the expected quantity and quality of discharge from each sediment control pond.

If seasonal variation of the quality of the discharge is anticipated, then a detailed description of that seasonality, along with a prediction of the effluent quality, is required.  If site specific water quality objectives are desired for the project, the application should include recommendations on your proposed site specific objectives.   The development of any site specific water quality objectives needs to be supported by a detailed evaluation and discussion of the background water quality and hydrology.

Detail on the receiving environment is required.  The receiving environment assessment should include:

- description of the initial dilution zone for each discharge point and an assessment of attainment of BC water quality guidelines at the edge of the IDZ.

- distances to downstream features and description of any potential downstream users,

- an overview of the aquatic resources along with an evaluation of the expected potential impacts from the discharge to those resources, and

- discussion on background water quality in support of any site specific water quality objectives that may be desired for the site.

A detailed sampling plan is required.  The plan must:

- identify effluent, background, and receiving environment sampling locations,

- outline what samples and field parameters will be collected and the frequency of sampling to be undertaken for each sampling location and parameter.

- outline procedures to develop a regression analysis to correlate TSS to turbidity to allow for in-situ monitoring.  This may have to be done for the different types of sediment expected to be encountered and each probe used, as the relation may be probe specific.

- include GPS coordinates for all receiving environment, background and effluent sampling sites that will be used in the sampling program.

The application needs to include details regarding any settling agents that may be used, including:

- type, where and how the agent will be added,

- method of mixing/conditioning required,

- collection of settled particles,

- description of maintenance of settling areas and  frequency of inspections, and

- monitoring of water quality that will occur during the use of these agents.

A Sediment and Erosion Control Plan needs to be provided.   The plan should be designed to function as a stand alone field document for the environmental monitor and contractors to use.  The document should:

- identify and reference appropriate installation and maintenance of sediment control measures,

- outline site resources required to successfully implement the plan,

- identify reserve resources to be kept on site for emergency erosion control,

- identify opportunities and methods for short term erosion control on slopes,

- identify preliminary locations for implementing erosion control measures and settling ponds, identify the type and source of erosion control rock.

The application should provide contingency plans for sediment control and spills and should include:

- the potential need for additional pond capacity or additional pond construction,

- methods for adjusting  retention time due to accumulated volume, turbulence and currents in the ponds and,

- action levels or triggers where contingencies or additional measures will be employed and a description of what those additional measures will involve, and

- spill response plans – please refer to BC guidelines for Industrial Emergency Response plans.

An Environmental Monitoring Plan needs to be submitted.  The plan should:

- identify the environmental monitor for the project and detail appropriate affiliations, qualifications and experience,

- outline the equipment required for the environmental monitoring program,

- outline reporting requirements for the environmental monitoring including the frequency and distribution.  Environmental monitoring reports should include an description of current on site activities and issues, provide an overview of upcoming activities and issues, and summarize water quality data collected and the environmental protection measures implemented.

Example introductory wording for an Erosion and Sedimentation Control Plan:

Objectives

The primary objective of this Erosion and Sedimentation Control Plan (ESCP) is to prescribe general and site-specific Best Management Practices (BMPs) that reduce the potential for impacting the surface water quality in watercourses downstream of mining related operations by minimizing surface erosion and sediment generation. The ESCP provides a summary of the site locations and associated erosion and sediment control issues, identifies site-specific objectives for BMPs, and provides BMPs that may be appropriate for each site location.

Strategies

The following strategies for achieving the above objectives will be described.

- Discuss the overall approach for controlling site erosion,

- Identify the runoff, erosion and sediment control issues associated with construction,

- Discuss erosion and sediment BMPs that may be suitable,

- Prescribe general and site-specific best management practices (BMPs) that reduce the potential for impacting the surface water quality in watercourses downstream of the site and

- Monitor, maintain and evaluate the success of the ESCP.

## 4.4 Fuel Management Plan

The Fuel Management Plan outlines the handling, dispensing and storage facilities and equipment, and procedures to be followed at the minesite during construction and operation.

## 4.5 Waste Management Plan

Waste management strategies to be followed during construction, operation and closure will be briefly described in the MA/EMA Applications whereas a detailed plan will be in the Appended EMP. The WMP will clearly outline all discharges through the various construction and operation phases of the project.

- Air contaminants – list sources, open burning, incineration, and include spills, dust, fugitive emissions from all processes (including cooling), emissions from ponds and yards, emissions from electrical generation. (Depending on the mining process, there may be need for an air discharge permit.) Refer to information box: Open Burning.

- Effluents – list sources, include spills, exfiltration, spray irrigation other losses from processes including cooling, sewage (refer to information box: Sewage Discharges), and storm water discharges.

- Refuse – list sources, include spills and other losses of materials including leachate, materials from landfilling or land-farming or recyclable materials. Refer to information box: Refuse.

- Site Contamination – an inventory of known or suspected site contamination and the potential for further soil or groundwater contamination on or near the site; and proposed site decommissioning or planned site remedial activities including information required for the completion of a site profile as described in the Contaminated Sites Regulation, at http://www.bclaws.ca/EPLibraries/bclaws_new/document/ID/freeside/375_96_00

| Open Burning Open burning of land clearing debris shall be carried out in accordance with the Open Burning Smoke Control Regulation. Open burning of construction wastes and other wood waste requires authorization under EMA. Please contact the regional Environmental Protection office for more information about applying for an authorization Refuse The Ministry’s policy for disposal of putrescible wastes (food wastes) for exploration, construction and industrial camps is incineration in a properly designed auxiliary fuel-fired refuse incinerator. For operations under 100 persons, a permit is not required for refuse disposal. For operations over 100 persons, an application under the EMA is required. Please visit the following website for additional information: http://www.env.gov.bc.ca/epd/waste_discharge_auth/intro.htm Select Wastes Landfills – The ministry’s policy is to recycle as much as possible from construction sites. The operation of a landfill at a mine site requires an authorization under the EMA. The “Landfill Criteria for Municipal Solid Waste” provides information on the requirements for siting, design, operation and closure of landfills in BC. http://www.env.gov.bc.ca/epd/mun-waste/waste-solid/landfills/ Please contact the regional Environmental Protection office for more information about applying for an authorization. | Sewage Discharges For sewage discharges greater than 100 persons, or where discharge is to surface waters, registration under the Municipal Sewage Regulation is required. More information on registering under the MSR can be found at: http://www.env.gov.bc.ca/epd/mun-waste/regs/msr/ Some key points to consider: - A pre-registration meeting with the Regional Manager, Environmental Protection, is recommended at least 60 days prior to registration. - Registration of a discharge under the MSR must occur at least 90 days prior to a discharge from the municipal sewage treatment system and must be accompanied by a $200 registration fee. - Completion of the environmental impact study and the operating plan for the facility must be completed 90 days prior to discharge from the system. - The regulation requires the sewage facility to be classified under the Environmental Operators Certification Program (EOCP), and that the facility by operated and maintained by persons certified in accordance with the EOCP or by an alternative training program that is approved by the director. |
| --- | --- |

## 4.6 Chemicals and Materials Storage and Handling Plan

This section will identify potential chemicals and substances that are classified as/or are deemed to be potentially hazardous (including toxic) to be used during construction and during operation.  This subsection will also briefly describe storage and handling plan for the identified chemicals and substances.

## 4.7 Wildlife Management Plan

The goal of the Wildlife Management Plan will be to minimize impacts on wildlife in the project area, with particular reference to focal species of interest and species at risk.  It will provide details of the activities directed to that goal including best management practices during construction and operation, employee education requirements and programs as related to those practices, detailed monitoring and reporting plans, provision for adaptive mitigation, ongoing consultation with stakeholders, and preliminary recommendations for consideration during reclamation.

## 4.8 Health and Safety

Health and safety aspects of the project will be described in this subsection. This will include issues in respect of water supply and treatment, sewage treatment and handling, air quality, explosives storage and handling, and occupational health and personal safety.

## 4.9 Traffic Management Plan

This will include general description of general policy for mine site haulage roads and the access road to the mine site.  This will further include measures to be followed on the concentrate haulage road(s).

## 4.10 Emergency Response Plan

This section will present the Emergency Response Plan in accordance with regulatory requirements including a Spill Response Plan for  prevention and management of spills and fugitive emissions onsite and on product transportation routes.

- A contingency plan should be developed for preventing, minimizing or containing emergencies. The contingency plan should also include plans for process upsets and non-compliant discharges (i.e. collection ponds with pump back systems; back up treatment systems). Guidance for contingency planning is provided in:

  - Guidelines for Industry Emergency Response Plans, at http://www.env.gov.bc.ca/epd/hazwaste/guideline_docs/index.htm

  - CAN/CSA-Z731-M91 Emergency Planning for Industry published by the Canadian Standards Association, at http://www.csa.ca

  - Spill Reporting Regulation BC Reg. 263/90 at http://www.qp.gov.bc.ca/statreg/reg/E/EnvMgmt/263_90.htm

  - Environment Canada links to regulations and guidelines for Emergency Planning under CEPA, at http://www.ec.gc.ca/ee-ue/default.asp?lang=En&n=E3A506F8-1

## 4.11 Archaeological Impact Mitigation and Management Plan

This will include:

- Archaeological and Cultural Heritage Resources Awareness Training

- Training for Archaeological Monitoring

- Detailed Chance Find Procedures including obtaining required permits

- Applicable Legislation, Regulations and Guidelines

- Heritage Inspection and Alteration Permits

- Protection of existing sites

# 5. RECLAMATION PROGRAM

## 5.1 End Land Use Objectives

Determination of the target species for land capability interpretations and end land use objectives will be made by the proponent in consultation with provincial government staff and First Nations.

## 5.2 Soil Handling Plan

The soil handling plan will present the soil salvage, stockpiling, and replacement strategy.  Soil salvage, storage and replacement based on information collected in accordance with section 2.7 will be described in detail. A map will be provided showing soil ratings and proposed salvage depths.

Salvage requirements will include:

- descriptions of the soils (or suitable overburden) to be salvaged;

- identification of materials or layers to be stripped separately and how to operationally distinguish them;

- total depth to be salvaged;

- equipment to be used and constraints (e.g. wet conditions) on stripping;

- erosion control and sediment retention measures required for exposed surfaces;

- anticipated volumes of each soil type;

- proposed training or supervision of operators by a soil scientist; and

- measures to protect the soil resource.

Stockpile requirements will include:

- description of soil stockpile locations, volumes depths and anticipated storage times;

- identification of layers or materials to be stored separately, and justification for doing so;

- storage requirements, including erosion control and sediment retention;

- descriptions of stockpile treatments during the storage period; and

- topsoil and other material identified in the soil survey which is to be salvaged as a growth medium stored in pre-determined locations, marked and identified accordingly on the soil handling plan.

The soil replacement strategy will address:

- timing and sequencing of soil salvage and replacement;

- methods of soil replacement;

- timing and methods of soil replacement, including equipment to be used, materials and depths to be replaced on a facility specific basis;

- requirements for amendments such as fertilizer and limestone; and

- methods to monitor/alleviate compaction during and following re-application.

## 5.3 Environmental Monitoring and Surveillance

A detailed description of the means by which environmental protection and quality control will be achieved during all stages of reclamation, particularly soils salvage, soil replacement and any requirements pertaining to Metal Leaching and Acid Rock Drainage (ML/ARD). This will include details of the authority and reporting sequence of any environmental staff and the procedure for providing reports and updates to government agency representatives, particularly the Mines Branch and MOE.

## 5.4 Conceptual Final Reclamation Plan

A conceptual final reclamation plan for the closure or abandonment of the mining operation will be provided. This plan will be in reference to, and consistent with, Parts 10.5 and 10.6 of the Code. The following minimum requirements for inclusion in the plan will be addressed.

## 5.5 End Land Use Objectives

End land use objectives for the minesite will be clearly identified and mapped. The map will overlay the pit, tailings impoundment, waste rock dumps, and any other facilities to remain following closure. Target ecosites will be identified. A general description of how the proposed reclamation program will achieve the end land use objectives will be provided. These objectives will be clearly described in the reclamation and mine plans, and will form an integral part of operations throughout the mine life.

## 5.6 Productivity or Capability Objectives

Productivity or capability objectives, how they will be achieved, and how reclamation success will be measured, will be provided for each of the specified end land use objectives.

## 5.7 Long-term Stability

Long-term stability, both physical and chemical, will be adequately addressed for all structures and discharges from the minesite. This will include consideration of future erosion, creep, mass wasting, and compatibility of final land forms with the surrounding landscape.

## 5.8 Treatment of Structures and Equipment

A description of structures and/or equipment to remain in place following mine decommissioning, and proposed reclamation treatments will be provided.

## 5.9 Waste Dump Reclamation

Proposed waste rock dump reclamation will be described in detail, including anticipated final configurations, proposed resloping, post-closure water management, surface treatment to alleviate compaction, details of soil replacement, a description of proposed revegetation methods, and trace element uptake in vegetation. Conceptual post-mine cross-sections will be provided along with a map illustrating section locations. Waste dumps will be optimized for snow/water retention (where appropriate), habitat diversity and aesthetic consistency with the adjacent landscape.

## 5.10 Tailings Reclamation

Proposed tailings reclamation will be described in detail, including anticipated final impoundment configuration, any proposed resloping, post-closure water management, including a spillway.  Methods of soil replacement on tailings dam faces and a description of proposed revegetation methods will be provided. Concerns related to trace element uptake in vegetation will be addressed. Long-term stability and maintenance requirements will be discussed.

## 5.11 Pit Reclamation

The plan will describe whether or not the pit will be flooded at closure and if so, details of water quality and any discharges to the receiving environment will be provided. Details of reclamation / revegetation measures to be undertaken within pit areas will be provided along with geotechnical stability assessments required for flooding proposals.

## 5.12 Watercourse Reclamation

Details of the re-establishment of post-mine watercourses and minesite water management will be provided. The plan will detail the volumes of water that will potentially be affected by ML/ARD or other mine effluents, and how they will be collected and treated to meet discharge water quality criteria. Similar detail for diversion systems of clean water that do not require collection and treatment will be provided. Long term operations and maintenance requirements will be provided.

## 5.13 Road Reclamation

Roads will be revegetated where applicable (i.e. most locations unless there is a specific elevation, terrain or other exemption agreed to by the Chief Inspector) and decommissioned to ensure geotechnical and hydraulic stability.

## 5.14 Trace Element in Soils and Uptake in Vegetation

The plan will outline a proposed program to assess trace element uptake in soils and vegetation at mine closure, and where possible, during the mine life.

## 5.15 Disposal of Toxic Chemicals

A list of chemicals or reagents to be used on site, and how these will be managed at closure will be provided.

## 5.16 Contaminated Site Requirements

Submit site profiles and describe site investigations conducted according to the Contaminated Sites Regulation.

## 5.17 Temporary Closure Care and Maintenance and Monitoring

Potential temporary or short-term closure required Care and Maintenance and monitoring that may be required for geotechnical, ML/ARD, revegetation, sedimentation or other requirements depending on the site and closure scenario will be described.

## 5.18 Operational and Post Closure Monitoring

Potential long-term monitoring that may be required for geotechnical, ML/ARD, revegetation, sedimentation or other requirements depending upon the site and closure plan will be described.

## 5.19 Detailed Five Year Mine Reclamation Plan

A conceptual mine plan for the next five years of operation will be provided. Content requirements of the mine plan will be determined on a project specific basis between the Mines Branch and RCDC.

## 5.20 Conceptual Life of Mine Reclamation Plan

A conceptual final reclamation program, including research that will be required in order to finalize the plan, will be provided. The program will be summarized in a tabular form and mapped.

## 5.20.1 Information sources used to develop revegetation plan will include:

- Regional plant succession patterns;

- Final site productivity/species objectives;

- Soil characteristics; and

- Reviewing results of other reclamation work in the area.

The reclamation plan will include information on vegetation development, supported by information on soil conditions, land use objectives, natural plant succession and the planned species.

## 5.20.2 Closing of Wells

Address how and when wells will be closed (e.g. water supply, monitoring, remediation, dewatering, geotechnical boreholes, test pits). Closure should be done in accordance with requirements under the Water Act and minimum requirements in the Ground Water Protection Regulation.

# 6. METAL LEACHING / ACID ROCK DRAINAGE (ML/ARD) ASSESSMENT

Précis

This section will provide a synopsis of ML/ARD assessment and prediction of mine drainage chemistry completed for the Environmental Assessment providing cross references hyperlinks to detailed information in the EA Application.  The section will also provide all additional information available from the continued ongoing testworks related to ML/ARD.

The MA/EMA applications will include the following:

## 6.1 ML/ARD Prediction and Prevention Plan

In addition to a synopsis of ML/ARD assessment and prediction of mine drainage chemistry completed for the Environmental Assessment the Mines Act Permit application, will provide updated ML/ARD data that include / reflect results of testwork that has been completed subsequent to what was provided in the EA Certificate application assessment.

The program will describe the geochemistry of all materials to be disturbed or created during mining.  Materials at the Red Chris Project that have been/will continue to be investigated for ML/ARD potential includes overburden, waste rock, ore and low grade ore and tailings.  Mine components of the Project that have been/will continue to be assessed for ML/ARD potential include the pit walls, waste rock dumps, low grade ore and ore stockpiles, tailings impoundment, borrow materials, plant site and roads (note: modify mine materials and components as appropriate). The plan will,

- provide waste rock, tailings and low grade ore characterization, volumes, mine sequencing, segregation/disposal methods, mitigation/management plans, contingency plans, and concepts for operational and post-closure monitoring and maintenance plans,

- clearly delineate geochemical source terms and methods utilized in geochemical predictive modeling (including the use of any relevant geochemical analogues) and present them in a clear and transparent manner,

- assess the lag time to ML/ARD onset for all potentially acid generating (PAG) materials and utilize this information in the development of management plans, and.

- for waste rock segregation, the application will assess the feasibility to successfully segregate PAG and non-PAG waste (NAG) materials during operations, propose geochemical segregation criteria and identify operational methods that will be required to achieve geochemical characterization during operations (i.e. geochemical surrogates, on site lab, procedures needed etc); include a sensitivity analysis to assess the effects of imperfect segregation of waste rock, and

- ML/ARD prevention/management strategies under a temporary closure or early permanent closure scenario, including waste rock, ore stockpiles, low grade ore, pit water quality

## 6.2 Materials Handling Plan

A Materials Handling Plan will be developed as an integral component of the ML/ARD Prediction and Prevention Plan and will include day-to-day operational management and materials handling procedures designed to integrate the ARD Prediction and Prevention Plan with the disposition of excavated, mined and processed material. The components of the plan will include:

- Identification and classification of PAG rock prior to extraction;

- Implementation of the PAG rock management strategy;

- Record keeping to provide an auditable trail of:

  - all sampling and testing used to identify and classify PAG and NAG rock from the mine

  - to record the location of when and where all PAG and NAG rock is placed

- Collection of data to improve predictions relating to onset of ARD and release of contaminants in waste rock dump and pit wall drainage

- Short and long-term control and management of drainage from PAG rock dumps.

## 6.3 Mine Waste Sequencing and Waste Placement Plan

(for Waste Rock and Low Grade Ore)

This Plan will include:

- Scheduling

- Placement methods

- Geochemical characteristics

- Operational management strategies

- Operational monitoring methods

## 6.4 Mitigation Plans

The Plans will include:

- Prevention works

- Disposal needs

- Treatment and control proposals

- Contingency plans

These Plans will include:

- For  water covers proposed for ML/ARD management, information will be provided to identify the types, volumes and geochemistry of mine waste to be flooded, the lag time to onset of ML/ARD in mine waste to be flooded, the disposal methods and location, the time until full flooding will occur, information to demonstrate that mine wastes will remain flooded during extreme climatic events, measures to mitigate soluble contaminants that could affect water quality, an assessment of geochemical stability under flooded conditions, and monitoring and maintenance requirements to ensure geochemical and physical security of flooded mine wastes (refer also to ML/ARD guidelines),

- For engineered covers proposed as a ML/ARD mitigation plan for the project, a conceptual design will be provided including the design objectives and principles, the characteristics and volumes of cover materials required, construction methods, assessment of expected performance and long-term effectiveness under the expected range of climatic conditions, monitoring and maintenance requirements, contingency plans and costs of constructing and long-term monitoring and maintenance (refer also to ML/ARD guidelines),

- For drainage collection and treatment mitigation strategies, a conceptual design will be provided including location, characterization of influent and effluent chemistry and flow, demonstration of the effectiveness of the drainage collection and holding system, conceptual design information on the treatment process, predicted reagent use, assessed performance under the expected range of flow and climatic conditions, sludge disposal plan, the operating, monitoring and maintenance requirements to ensure successful treatment is sufficient to achieve long-term environmental protection requirements, and capital and operating costs (refer also to ML/ARD guidelines)

## 6.5 Water Quantity and Quality Monitoring Plan

This Plan will include:

- Procedures for determining the mine drainage quantity

- Procedures for determining the mine drainage quality

- Procedures for determining the effectiveness of drainage diversion and collection structures

- Procedures for and verification of pit flooding predictions

The Plan description will include:

- The application will integrate results of the ML/ARD prediction work and surface and groundwater hydrology and water balance information to develop water quality predictions that will be used as a basis for impact assessment and to determine materials handling procedures and to assess and develop ML/ARD mitigation/management requirements for the project.  Geochemical modeling will be presented in a clear and transparent manner and the assumptions and rationale used to estimate water quality will be thoroughly explained.

- Information describing how current baseline and ongoing surface and groundwater quality and flow rates are anticipated to be altered by individual mine components will be provided. Information will focus particularly on the open pit, ore and low-grade ore stock piles, waste rock pile, and tailings/waste rock impoundment,

- and how to prevent erosion and sediment discharge during the construction, operational and closure phases will be provided.

# Preamble to Sections 7, 8 & 9 for Two-Step EMA Permitting

The EMA permit application can seek authorization for a major mine’s construction stage (sediment control and blasting residues) and operations stage (metal/chemical residues and contact water and tailings). Alternatively, the proponent may take a two step approach to the EMA permit by seeking authorization for only the construction stage and deferring authorization for the operations stage until engineering is sufficiently detailed to support an application for amendment to the permit.

If the application will follow a two-step EMA permitting approach then the level of detail required in sections 7, 8 & 9 will only need to provide details for matters such as the management of storm water during the site disturbance and construction stage. Although detailed engineering about the operations stage may not be available to enable conditions to be set for tailings disposal and discharges of mine water (seepage, pit, underground and excess process water), the initial permit application should still include high level informant about what is intended for the operations stage.

# 7. DISCHARGES AND TREATMENT

Characterize the quantity and quality of all discharges as outlined on the application form Part 3 Discharge Details, including:

- A description of methods for determining the quantity and quality of the effluent, air emissions and solid-waste pollution-control works and sources of discharge (i.e. bench scale tests; pilot plant results; manufacturer’s design specifications and performance guarantees, etc.).

- Tracking of all inputs at each stage in the process through to output, especially identifying all contaminants produced at each stage of the process (chemicals and reagents used; contaminants liberated or created, etc.)

- Describe how the pollution control works are properly designed and are appropriate for the intended use. The ministry expects applicants will select works or systems adequate to meet or exceed the statutory requirements and industry standards for preventing or minimizing adverse public health and environmental impact. The evaluation should consider capital and operating cost, design capacity, effectiveness, reliability and weaknesses (dealing with issues such as fires, power outage, flooding, etc.), waste products, maintenance and personnel training. Identify alternatives and pollution prevention approaches that have been assessed.

# 8. ENVIRONMENTAL EFFECTS ASSESSMENT FOR EMA

## 8.1 General

The applicant must provide an assessment of the overall risks of this development and the discharges to aquatic, terrestrial and atmospheric resources. The assessment typically uses a watershed or airshed, and land-use management approach. It includes an identification of critical biological receptors, in each of the environmental media (air, water, and land) which are, or could be, impacted by losses from the operations. This assessment should include:

- Comparison of proposed discharge quality to known discharge criteria, guidelines or industry practices;

- modeling work performed to project the emissions into the receiving environment;

- predicted incremental increases in relevant ambient parameters;

- cumulative impact when the incremental increase is added to the existing receiving environment levels; and

- potential impacts to humans and other receptors by comparison of predicted ambient environmental quality to:

    - established provincial or federal ambient environmental quality guidelines

    - comparison to other relevant scientific studies or sensitivity estimates of the human and nonhuman biotic components of the environment and the potential for acute, chronic or episodic damage and cumulative and synergistic effects

    - the use of human health and ecological risk assessment exposure models are one way for conducting a detailed impact analysis, considering the following guidelines:

    - Recommended Guidance and Checklist for Tier 1 Ecological Risk Assessment of Contaminated Sites in British Columbia, which can be obtained at: http://www.env.gov.bc.ca/epd/epdpa/contam_sites/policy_procedure_protocol/protocols/tier_1/index.html

      - Qualitative Human Health Assessment – Phase 1 Review of Methods and Framework Recommendations, at: http://www.env.gov.bc.ca/epd/epdpa/contam_sites/guidance/external/humanhealthreport.html

      - Risk Assessment Guidance for Superfund (RAGS) Part A http://www.epa.gov/oswer/riskassessment/ragsa/index.htm

## 8.2 Areas of Specific Topics

Discussion may include but will not be limited to the following.

### 8.2.1 Environmental Effects Assessment

The report should identify critical effects thresholds, triggers or what would be considered a biologically significant change for the various EEM components (water, sediment, benthic invertebrates, fish, etc.). These thresholds are flexible guidelines and are designed to allow adaptive management of discharges.

### 8.2.2 Fisheries and Aquatic Resources

- This section requires a discussion of sensitive fish habitats (spawning, rearing, overwintering, etc) that may be exposed to some degree of mine-related effluent.  Evaluate options to determine what acceptable effects will occur.

- Fish populations – what types of fish will be affected? What fish will be around the site?

- Interpretation and statistical analysis of available data on EEM is required.

- Interpretation of biological monitoring tools (benthos, periphyton, etc.) that will be used in the ongoing impact assessment is required.

- Establish the probability that mine related impacts to valued resources may occur as a result of exposure to effluent both with and without pollution control works.

- Consider the requirements of the Federal EEM.

### 8.2.3 Impacts to humans

Describe impacts to humans if applicable.

### 8.2.4 Surface Water and Groundwater Hydrology

Describe potential impacts to local surface and groundwater hydrology from all discharges from mining and waste disposal. Discuss their significance. Identify uncertainties and how they would be addressed in the monitoring efforts.  Consider impacts from diversion of storm water around mine working to the receiving environment, and changes to flows downstream of mine workings that may be impacted from water storage on site.  The review should also consider changes to the groundwater regime around major mine infrastructure (pits/tailings impoundment.

### 8.2.5 Water Quality Modeling

- A prediction of water quality will be made for major mine components (waste rock dumps, open pit, low grade ore stockpile, ore stockpiles, tailings impoundment etc.) and all site surface water discharges and groundwater seepages for key times (i.e. operations, closure, post-closure).  This assessment will include volumes, water quality, discharge structures and location, potential impacts on the receiving environment and the description of any mitigation strategies and/or treatment processes required to prevent/manage ML/ARD.  It will describe contingency plans if there are significant uncertainties or risks associated with the predicted water quality, and for dealing with excessive run-off events and drought conditions if necessary,

- Include details around all assumptions made, inputs into water quality and groundwater models, all data used for building model, maps/diagrams showing where predictions are made/applicable, dilution estimates for each receiving environment prediction point, with consideration of low flow conditions or conditions during critical aquatic habitat windows.  Modeling results should be compared to BC guidelines.

# 9. PROPOSED DISCHARGE AND ENVIRONMENTAL EFFECTS MONITORING FOR EMA

- The applicant must propose a discharge monitoring program, using physical, chemical and biological indicators, to track the quantity and quality of all the emissions, effluent and solid waste discharges. This proposal must include proposed analytical methods and the QA/QC program. The program must conform to the standards specified in the British Columbia Field Sampling Manual:2003 (or a later edition), the Stationary Air Emissions Testing Manual, and the Environmental Data Quality Assurance Regulation, all available at. http://www.env.gov.bc.ca/air/wamr.

- Analysis must be performed using standard analytical methods, as specified in the most recent edition of the British Columbia Environmental Laboratory Manual and supplements to the manual, available at http://www.env.gov.bc.ca/air/wamr/.

- The applicant’s monitoring program must also provide details for the methods of collection of data, archiving of records, processing and reporting.

- The monitoring program should focus on the effluent discharges and collect data in the receiving environment at the Initial Dilution Zone (IDZ) (near field) and at mid and far field sites in order to determine the duration, extent and or severity of potential mine-related effects to water quality.

- The monitoring program will require an environmental effects monitoring program to track the impact of the discharges on the receiving environment. Where appropriate, this should dovetail with federal requirements (such as Metal Mine Effluent Regulations) and other joint programs such as airshed planning processes.

# 10. RECLAMATION COST ESTIMATES

This section will describe methods to be used to determine the reclamation cost estimate as required for various phases of mine development and closure. These cost estimates will form the basis of the timing and size of securities required as a condition of the Permit.

## 10.1 Post Closure Monitoring

A brief description of what items of work will be considered for post closure monitoring will be provided. This may include:

- Inspections

- Environmental monitoring

- Periodic reporting of inspection and monitoring work will be stipulated including annual dam safety inspection reports for tailings impoundments

## 10.2 Post Closure Maintenance (excluding water treatment)

- Cover Maintenance

- Spillway Maintenance

- This may also include periodic road maintenance, diversion ditch maintenance, slope remediation on dams and waste dumps, and geotechnical instrumentation repair or replacement

- Closure of all wells.

## 10.3 Post-Closure Maintenance (Water Treatment)

- Construction and operation of water treatment facilities

# 11. OTHER COMMENTS

Précis

This section will provide any other comments or references that may be necessary for the completeness of the Application.

## 11.1 Referencing to Earlier Submitted Information

- All key information as a brief summary against each section or subsection in the Table of Contents will be provided.

- For further details on the above information a hyper-link to a corresponding area in the earlier submitted documents such as the Environmental Assessment Application and Supporting Documents will be provided.

- The above linked information will be provided in a separate CD/DVD.

- All new information relevant to any section or sub-section will be provided in the MAPA.

## 11.2 References

A list of all references that will be used in the detailed facility designs, including all design guidelines, references and manuals will be provided.