# TCC 2021 LANDSCAPE STEWARDSHIP CHECKLIST

(To identify potential issues of a proposed development with the BCTS TCC Area FSP and other consultative commitments)

| Prepared by | Nona Phillips Forestry Consulting | Date | July 27, 2021 |
| --- | --- | --- | --- |

| TSL Block/Road Name1 | UBI / URI Number2 | Planning Recce # 3 | Est. Gross Area (ha)4 | Est. Gross Volume (m3)5 | Road length (m)6 | “Green” Stand Harvest7 (Y/N) | Stand Salvage Harvest8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TA1918_CAR-153 | Not assigned |  | 95.6 | 47,776 |  | Y | N |
| TA1918_CAR-155 | Not assigned |  | 42.2 | 21,127 |  | Y | N |
| TA1918_CAR-160 | Not assigned |  | 16.4 | 4,000 |  | Y | N |
| TA1918_CAR-162 | Not assigned |  | 62.5 | 17,184 |  | Y | N |
| TA1918_CAR-176 | Not assigned |  | 25.5 | 6,365 |  | Y | N |
| TA1918_CAR-177 | Not assigned |  | 14.1 | 3,886 |  | Y | N |
| TA1918_CAR-178 | Not assigned |  | 47.6 | 13,084 |  | Y | N |
| TA1918_CAR-180 | Not assigned |  | 29.5 | 9,506 |  | Y | N |

Note: each individual block or road that is included in the landscape level check should be included as row item in the landscape checklist

1 TSL Block: if current multiphase eg. TA640/Blk 1 / Road naming convention follows Table 26: Acceptable Road Naming Convention from BCTS GIS Data Standard. www.for.gov.bc.ca - /ftp/TCC/external/!publish/Data_Standards/ArcGIS/Multiphase/Standard/

2 If available for blocks currently under multiphase

3 If available eg. (QU-105a)

4 BAI = Best available info not required for roads

5 BAI = Best available info not required for roads

6 Only for the access road sections to the block boundaries

7“Green” Stand is a stand that has less than 35% dead trees (due to insect, fire or windthrow. Conifer trees with green and/or red needles are considered live trees and will be classified based on pathological indicators. Standing or windfall trees with grey or no needles will be considered dead trees; except for mountain pine beetle attacked lodgepole pine where a tree with less that 5.0% of “red” needles will be considered dead.

8 e.g.: burn, windthrow, Pine beetle, etc.

Shape File Location: R:\General_User_Workspace\TCC PLANNING\!Referrals_shapes\Quesnel (Williams Lake)> new naming convention, PDF map for checklist (template – block shapes/roads),  need to make sure it is linked to planned access, buffer shapes: roads 50m and blocks 100m  buffer for shapes

2020 FSP Objectives

# Sec. 5.1 Soils

| Permanent Access Structure Limits (FPPR, Sec. 36) and Key Road Issues1 | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Additional permanent access structures necessary to provide access beyond the cutblock or key road issues |  | X |
| Comments: |  |  |
| 1adjacent to private land or requires access to private land show the lot#, show rd. status for rd. tributary to new road (wilderness road, road permit etc.) |  |  |

# Sec.5.2 - Timber

| FPPR section 43.1 – Secondary Stand Structure | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Applies to “Targeted Pine leading Stands” within SBPS BEC Zone or FDU 1 West of the Nazko River & FDU 2 West of the Fraser River1 |  | X |
| Comments: |  |  |

# Sec.5.3 Wildlife

| Wildlife (General) (FSP, Sec.: 5.3.1 through 5.3.19) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development adjacent to or overlapping with any “Wildlife Habitat Areas” Notices or habitat for the species listed in the FSP Table 4 | X |  |
| GIS Analysis Identified & listed in the FSP Table 4 Species at Risk (SAR) & “Red Listed” species Mountain Caribou Grizzly bear Fisher Click or tap here to enter text. Note: - For the above listed species identified through GIS data analysis and from the FSP Table 4 (e.g.: American Badger, White Pelican, etc.), FSP Strategies 3b / field observation and 3.d. will apply and a QRP assessment.1 - For the above list of species record: if seen, or species used (i.e. dens, nests) or key habitat within the block or road, or the surrounding 25m of the boundary for a block or road a QRP field assessment is required. - When an observation of wildlife species or wildlife use, or key habitat needs for species are not observed in the field, a record that it was not observed must be provided and kept on the proposed development records. - GAR Order – Wildlife Habitat Areas link: https://www.env.gov.bc.ca/cgi-bin/apps/faw/wharesult.cgi?search=wlap_region&wlap=Cariboo |  |  |
| Comments: See sections 5.3.14 Grizzly Bear and 5.3.17 Fisher. The Route card identified that; - all blocks overlap CDC Non Sensitive Area -Mountain Cariboo and Internal Use Wildlife Sensitive Incident Observations -Mountain Caribou. - blocks 160, 162, 177, 178 and 180 are adjacent to (do not overlap) a Mountain Caribou WHA (no harvest) 5-106. - block 180 overlap Cariboo Telemetry (species unknow) - block 180 overlaps Internal Use Sensitive Survey Observation (species unknown) |  |  |

# Sec. 5.3.2 Moose

| High Value Moose Wetland Management Zone (HVMWMZ) |  |  |  |  |  | APPLIES | DOES NOT APPLY |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed development adjacent to a key wetland. |  |  |  |  |  |  | X |
| Block # | Within 200m *) | Within 500m *) | Within 1km *) of HVMW | Road Density within 1km *) | Wetland is < 6ha | Initial Thermal % | Initial Security % |
| Comments: |  |  |  |  |  |  |  |

| Moose Management Unit (MMU) (it’s different than HVMWMZ) |  |  |  | APPLIES | DOES NOT APPLY |
| --- | --- | --- | --- | --- | --- |
| Proposed development adjacent to a key wetland. |  |  |  |  | X |
| Block # | Within 100m*) | Wetland < 6ha | Initial Thermal % | Initial Security % |  |
| Comments: |  |  |  |  |  |
| Check wetland layer spatial for accuracy with the VRI. Provide documentation for thermal and security cover calculation. |  |  |  |  |  |

*) Slope distance

# Sec. 5.3.3 Mule Deer

| Mule Deer (within FDU 1 - Ungulate Winter Range U-5-001 & FDU 2 - Ungulate Winter Range U-5-002.) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development within or adjacent to Ungulate Winter Range (location of winter range objectives and wildlife measures http://www.env.gov.bc.ca/wld/frpa/uwr/approved_uwr.html) |  | X |
| Mule Deer Winter Range snowpack zone: Choose an item. |  |  |
| Mule Deer Winter Range habitat management zone: Choose an item. |  |  |
| Overlap with Mule Deer Winter Range topographical buffer: Choose an item. |  |  |
| Comments: |  |  |

# Sec. 5.3.4 Furbearers

| Furbearers (General) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Where harvesting removes > 50% of stand basal area in contiguous areas > 5 ha and within 100m of riparian area | X |  |
| Comments: The wetland associated with block 1C was compared to the ortho photos and they are similar. Block; Riparian Feature. 153; May be within 100m of an S4 stream (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground.. 155; May overlap the RMZ of an S4 stream (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground.. 160; May overlap the RMZ of an S4 and five S6 streams (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground.. 162; May overlap the RMZ of an S4 and S6 stream (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground.. 176; May overlap the RMZ and RRZ of an S3 stream (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground. (Also see section 5.3.14 Grizzly Bear Below). 177; No riparian features within 100m.. 178; May overlap the RMZ and RRZ of an S3 and two S4 streams (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground. (Also see section 5.3.14 Grizzly Bear Below). 180; May overlap the RMZ of an S4 stream, the RMZ of two S6 streams and within 100m of an S3 stream. (from the TCC Stream Class layer). The stream layer may not be accurate so the stream existence/location/class must be confirmed on the ground. (Also see section 5.3.14 Grizzly Bear Below). A field assessment / verification needed. Fisher habitat additional piles may be required. |

# Sec. 5.3.14 – Grizzly Bear

| Grizzly Bear Habitat | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development within or adjacent to grizzly bear habitat | X |  |
| Grizzly bear habitat capability areas: Moderate High Very High |  |  |
| Comments: All blocks overlap high grizzly bear capability |  |  |
| Grizzly bear habitat feature adjacent to: Stream S1, S2 and S3 Fish spawning reach |  |  |
| Grizzly bear habitat feature adjacent to critical fish habitat: Yes |  |  |
| Comments: Blocks 176, 178 and 180 are probably adjacent to an S3 stream but it is not known if the sections are spawning reaches. Block 180 is adjacent to critical fish habitat (salmon/bull trout). Blocks 155, 176 and 178 overlap critical fish habitat (salmon/bull trout). |  |  |

Sec. 5.3.17 – Fisher

| Fisher Habitat | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development adjacent to or overlapping known Fisher habitat | X |  |
| Blocks are located within Dry Forest Fisher Habitat Sub Boreal Moist Sub Boreal Dry List which types apply in table |  |  |
| Comments: A field assessment / verification needed. |  |  |

Sec. 5.3.18 and Sec. 5.3.19 Additional Species at Risk (SAR)

| Additional SAR under GAR (Listed in Table 7 of FSP) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development adjacent to or overlapping known species at risk. |  | X |
| GIS Analysis Identified & listed in the FSP Table 7 SAR & “Red Listed” species: Click or tap here to enter text. Click or tap here to enter text. Click or tap here to enter text. Click or tap here to enter text. |  |  |
| Comments: See section 5.3 Wildlife |  |  |

## 5.4 Riparian Areas

### 5.4.2 Streams, Wetlands and Lake Riparian Areas

| Riparian classification, Riparian Reserve Zone and Riparian Management Area | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within or adjacent to the RRZ or RMA of lake or wetland. |  | X |
| The Lake Class from GIS analysis is identified as: NA |  |  |
| The Wetland Class from GIS analysis is identified as: NA |  |  |
| Comments: |  |  |

| Lakes with Lake Management Class (FSP Tables 8 and 9) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within or adjacent to the management zone of a “Classified Lake”. |  | X |
| The Lake Class is: N/A |  |  |
| Calculate the amount of forest disturbed area within the lakeshore management zone. |  |  |
| Comments: N/A |  |  |
| Proposed development is within 2000 metres of a wilderness fisheries lake |  | X |
| Comments: N/A |  |  |

## Sec. 5.5 Watershed Hydrology

| Watershed Hydrology | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within the Key Watersheds. | X |  |
| Choose the Key Watershed identified: Cariboo River |  |  |
| Identify the Key Watershed Sub-basin: Blocks 176, 177, 178 and 180 are in the Seller Creek Sub-Basin. Blocks 153, 155, 160 and 162 are in the Quesnel River Residual area. |  |  |
| The hydrological assessment for the Key Watershed sub-basin is From the June 2021 Forsite ECA report: Quesnel River Residual 0-10% Seller Creek 0-10% |  |  |
| Provide supporting LBR report date. G:\BCTS\Planning\Land Base Reporting - LBR |  |  |
| Comments (what is ECA %?): See above |  |  |

## Sec. 5.6 Critical Fish Habitat

| Critical Fish Habitat | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within 500m or adjacent to Critical Fish Habitat. | X |  |
| Comments: Block 180 is within 500 m of critical fish habitat (salmon/bull trout). Blocks 153, 155, 176 and 178 overlap critical fish habitat (salmon/bull trout). Note: If Grizzly habitat see FSP Sec. 5.3.14, strategy 3(c) |  |  |

## Sec. 5.7 Blackwater Quality Fisheries Resource

| Blackwater Quality Fisheries Resource | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within or adjacent to one or more of the Blackwater Quality Fisheries Resource |  | X |
| Comments: |  |  |

## Sec. 5.8 Salmon Watersheds

| Salmon Watersheds | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within or adjacent to one or more of the following Salmon Watersheds: Cariboo River | X |  |
| Proposed development is within 500m of the Fraser River or Chilcotin River No |  |  |
| Proposed development overlaps with terrain class: The following shows block overlaps with unstable and potentially unstable terrain: Block; Terrain Category. 153; Potentially Unstable. 155; Potentially Unstable & Unstable. 160; NA. 162; NA. 176; Potentially Unstable. 177; Potentially Unstable. 178; Potentially Unstable. 180; Unstable. |
| Requires terrain stability assessment: Yes |  |  |
| Comments: All blocks in this TSL overlap the Cariboo River Salmon Watershed. |  |  |

## Sec. 5.9 Fisheries Sensitive Watersheds

| Fisheries Sensitive Watershed (FSW) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within or adjacent to one or more of the Horsefly River Fisheries Sensitive Watershed. |  | X |
| Proposed development requires a Fisheries Sensitive Watersheds assessment Choose an item. |  |  |
| Active Fluvial Unit (AFU) along fish-bearing stream and direct tributaries to fish bearing streams (based on an aerial overview or a field assessment needed) |  | X |
| Comments |  |  |

## 5.10 Water in Community Watersheds & Licensed Waterworks

| Community Watersheds | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within a Community Watershed: N/A |  | X |
| Proposed development requires a Community Watersheds assessment NA |  |  |
| Licensed Waterworks: |  |  |
| Comments: |  |  |

## 5.11 Biodiversity

## 5.11.1 Seral Stage

| Proposed Development Area Seral Stage Targets |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Block # | Landscape Unit | Reporting Unit | Surplus MO (ha) | LBR Source Date: mm/yy |  |
| 153 | Lower Cariboo | ICHwk4 | 1,113 | June 2021 |  |
| 155 | Lower Cariboo | ICHwk4 | 1,113 | June 2021 |  |
| 160 | Lower Cariboo | ESSFwk1 | 7,410 | June 2021 |  |
| 162 | Lower Cariboo | ESSFwk1 (90%) | 7,410 | June 2021 |  |
|  |  | ICHwk4 (10%) | 1,113 | June 2021 |  |
| 176 | Lower Cariboo | ESSFwk1 | 7,410 | June 2021 |  |
| 177 | Lower Cariboo | ESSFwk1 | 7,410 | June 2021 |  |
| 178 | Lower Cariboo | ESSFwk1 | 7,410 | June 2021 |  |
| 180 | Lower Cariboo | ESSFwk1 | 7,410 | June 2021 |  |
| Comments: |  |  |  |  |  |
| Seral Drawdown / Criteria for a harvest to be met: |  |  |  | APPLIES | DOES NOT APPLY |
| Proposed development is consistent with the most recent Seral Stage Analysis for this Landscape Unit and Reporting Unit. (FSP Table 13 Mature + Old Seral Targets apply) |  |  |  | X |  |
| Does the proposed development have the MO seral availability (not applicable to Roads): Yes |  |  |  |  |  |
| Initial proposed criterion for Seral Drawdown: Choose an item. |  |  |  |  |  |
| Proposed development is within Criterion A: Choose an item. |  |  |  |  |  |
| If yes, provide recruitment area to offset Seral Drawdown, ensure its consistent with the recruitment area strategy in FSP section 5.11.1 Seral Stage. Provide recruitment area, VRI label, map, and shapefile, consult with Planning Forester: |  |  |  |  |  |
| Comments: All blocks are in units with surplus Mature+Old area. |  |  |  |  |  |

### 5.11.2 Spatial Distribution of Cutblocks, Landscape Connectivity and Species Composition

| Block | Adjacent other Openings (Y/N) | Patch Size |
| --- | --- | --- |
| 153 & 155 | Y | Very Large |
| 160 and 162 | Y | Very Large |
| 176 | Y | Very Large |
| 177, 178 & 180 | Y | Very Large |

| Patch Size |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Landscape Unit | NDT |  | Small Patches HO1 |  | Medium Patches HO | Large Patches HO | Recommendations from LBR report |  |
| Lower Cariboo | 1 |  | 0 |  | 0 | 818 | Group small and medium into large patches |  |
| Comments: All blocks are adjacent to other openings creating ones that are in the very large category. There is still room in the largest category. 1HO is Harvest Opportunity. |  |  |  |  |  |  |  |  |
| Proposed development is consistent with or trend towards the “Patch Size Target Ranges” outlined in FSP Table 15? Yes |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |
| Reference: Patch Size target ranges by NDT types, G:\BCTS\Planning\Land Base Reporting - LBR\LBR 2021June\Tables\20210622_Cariboo_LBA_Tables\Cariboo_Patch_and_Seral_Distribution_HarvestOpp_20210618.xlsx |  |  |  |  |  |  |  |  |
| NDT |  | Small |  | Medium |  | Large |  | Very Large |
| 1,2,3b,4 |  | <40 |  | 40-80 |  | 80-250 |  | 250+ |
| 3a |  | <40 |  | 40-250 |  | 250-1000 |  | 1000+ |

| Landscape Connectivity |  |  |
| --- | --- | --- |
| Is there an established Connectivity Corridor? Yes All blocks are adjacent to (do not overlap) a connectivity corridor mapped in the Total Chance Plan. |  |  |
| Is the existing Connectivity Corridor consistent with the Natural Connectivity Characteristics Frequency in FSP Table 16? Yes |  |  |
| If Connectivity Corridor does not exist or meet the objectives of FSP Table 16; establish Connectivity Corridor that maintains the natural connectivity characteristics in the area(s) of the proposed harvest: NDT: 1 BEC Unit: ESSFwk1 NDT: 1 BEC Unit: ICHwk2, ICHwk4 |  |  |
| Comments: |  |  |
| Species Composition |  |  |
| Key leading spruce stand located in one of the following CCLUP sub-unit management zones: Taseko Lake, Kluskus, Anahim Lake, Chezacut, Kleene Kleene, Eagle or Palmer Lake? No |  |  |
| Comments: |  |  |
| Key leading deciduous stands located in the Quesnel or Beaver Valley CCLUP sub-unit management zones? No |  |  |
| Comments: |  |  |
| Key leading aspen stands located in the Lower Blackwater CCLUP sub-unit management zone and Chilcotin Plateau (Taseko Lake, Kluskus, Anahim Lake, Chezacut, Kleene Kleene, Eagle or Palmer Lake CCLUP sub-unit management zones)? No |  |  |
| Comments: |  |  |
|  | APPLIES | DOES NOT APPLY |
| Areas of “key leading spruce” or “key leading deciduous” or “key leading aspen” stands, should demonstrate in the Site Plan how these stands were considered in the design of “Wildlife Tree Retention Areas” (WTRA) for the proposed development. (FSP Sec. 5.11.2 Result/Strategy 4) |  | X |

### 5.11.3 Old Growth Management Areas (OGMA)

| OGMA | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development overlaps or is adjacent to: Permanent OGMA-Static Permanent OGMA-Rotating Transitional OGMA | X |  |
| Primary forest activities can be authorised if one or more of the following criteria are met (refer to the FSP Sec. 5.11.3 Results / Strategies): - Permanent OGMA-Static: Choose an item - Permanent OGMA-Rotating: Choose an item - Transitional OGMA Choose an item Choose an item |  |  |
| Comments: Blocks 153 and 155 are adjacent (do not overlap) permanent OGMAs on their west boundaries. Block 176 is adjacent (does not overlap) a permanent OGMA on its east boundary. Blocks 177 is adjacent (does not overlap) a permanent OGMA on its north boundary. Block 178 is adjacent (does not overlap) permanent OGMAs on its north and transitional OGMA on its south boundaries |  |  |

5.11.4 Cariboo-Chilcotin Grassland Benchmark Area

| Grassland Benchmark | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development within or adjacent to the “Grassland Benchmark Area”. |  | X |
| Comments: |  |  |
| The existing “Grassland Benchmark Area” spatial dataset is consistent with the ortho photography location of the actual grassland location? No |  |  |
| Comments: |  |  |

5.11.5 Mature Birch Retention

| Mature Birch Retention | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development within or adjacent to the “Mature Birch Retention” within following landscape units: Lower Cariboo | X |  |
| Comments: Blocks 153 and 155 overlap mature birch stands. The VRI was checked to confirm the species composition (see birch map provided). Two of the stands that are in Mature Birch polygon CAR_27_4380 do not have birch according to VRI. |  |  |

### 5.11.6 Wildlife and Biodiversity – Stand Level

| Wildlife Tree Retention (WTR) Targets (FSP Appendix D or FSP Table 17) | WTR % | APPLIES | DOES NOT APPLY |
| --- | --- | --- | --- |
| Where harvesting removes > 50 percent of the pre-harvest merchantable stand basal area or where the harvest is part of a shelterwood silvicultural system, the minimum areas for WTR expressed targets set out in the FSP Appendix D or if FSP Results/Strategy 5 applies Table 17 targets will apply. | 10% for both units | X |  |
| Where harvesting removes < 50 percent of the merchantable pre-harvest basal area, ensure, to the extent practicable, that high value wildlife trees are retained up to the targets specified in the FSP Appendix D or if FSP Results/Strategy 5 applies Table 17 targets will apply. |  |  | X |
| Comments: N/A |  |  |  |

## 5.12 Visual Quality

| Scenic Areas | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development or access roads are within a scenic area with a Visual Quality Objective (VQO) polygon in a known scenic area. The visual quality objectives are Preservation Retention Partial Retention Modification Maximum Modification | X |  |
| Scenic Corridors or High Elevation Viewpoints |  | X |
| Known significant public viewpoint from Lake surfaces for a scenic area associated with a lake River channel for a scenic area associated with a river Existing tourism facilities and key tourist use areas Existing tourism operations as defined in Section 5.13.1 Tourism Points for highways deemed significant by a QRP Points for parks and backcountry areas deemed significant by a QRP Other viewpoints that are deemed significant by a QRP |  |  |
| Comments: Blocks 153, 155, 160 and 162 overlap a modification VQO polygon. A Viewshed Analysis in ArcGIS must be completed including map and spatial data. Planning forester will propose significant public viewpoints that will require validation in the field. |  |  |

## 5.13 Recreation

## 5.13.1 Tourism

| Commercial / Private Recreation Tenures | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Overlapping licensed commercial / private recreation tenure holders within 2 km of the planned development. | X |  |
| Comments: All blocks in this TSL overlap two commercial recreation licences. - Crown Land 5406691-Guided Nature Viewing - Crown Land 5406975 -Multiple Use |  |  |

## 5.13.2 Recreation Sites and Trails

| Interpretive Forest Sites, Recreation Sites and Recreation Trails | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Recreation sites and trails within 100 m of the planned development. (BCGW layer and appendix E) |  | X |
| What type NA |  |  |
| Comments: |  |  |

5.13.3 Backcountry

| Backcountry | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within a backcountry polygon. | X |  |
| Known clubs, & tourism operations in the proposed development area | X |  |
| Blocks 153 and 155 overlap a backcountry polygon, associated with the Cariboo River, along their west boundaries. |  |  |

### 5.13.4 Land Act Order Trails

| Buffered Trails | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development contains or is adjacent to a buffered trail. (maintain 50 metre management zones on either side and minimum basal area retention of 85%, except insect control, maintenance of recreational values and fire control…) | X |  |
| Yes |  |  |
| Comments: A snowmobile trail pass just inside the west boundaries of blocks 153 and 155. The trail parallels the Cariboo River. The mapped trail follows what seems to be a trail opening on the ortho photo (see map attached to the landscape checklist) |  |  |

5.13.5 Alexander MacKenzie Heritage Trail / Nuxalk-Carrier Grease Trail

| Alexander Mackenzie Heritage Trail | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within the 100m buffer zone of the trail. |  | X |
| Within the “viewscape” of the Alexander Mackenzie Heritage Trail. |  | X |
| Proposed development is consistent with the Trail Management Plan. |  | X |
| Comments: |  |  |

5.13.6 Wildcraft

| Wildcraft | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Does known Access Management Plan apply? |  | X |
| Comments: There is no known Government Access Management Plan for this area. Quesnel Lake SRDZ – 30% roaded access |  |  |

5.14 Cultural Heritage

5.14.1 Pine Mushrooms

| Pine Mushrooms Area | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within or adjacent to a known pine mushroom area. (Anahim Lake Integrated Resource Management Zone) |  | X |
| Comments: |  |  |

### 5.14.2 Community Areas of Special Concern (CASC)

| CASC (no-harvest areas) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is adjacent or within to CASC. (see exceptions in the FSP Sec. 5.14.2 Results / Strategies: insect control and interface fuel breaks) |  | X |
| Comments: |  |  |

### 5.14.3 Cultural Heritage Resources (CHR)

Comments: If required, a CHR mitigation strategy will be provided to the timber team following the referral period.

5.15 Grazing – Maintenance of Animal Unit Months

Maintenance of Animal Unit Months

Comments:

# 6.0 Measures

# 6.1 Natural Range Barriers

| Natural Range Barriers | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development overlaps or is adjacent within 200m of a natural range barrier. |  | X |
| Comments (after consultation with the District Range Agrologist): |  |  |

6.2 Invasive Plants

| Invasive Alien Plant Program | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development overlaps or is adjacent to an area identified by the “Invasive Alien Plant Program” (IAPP) or identified during primary field activities. |  | X |
| Comments |  |  |

Landscape Level Constraints Outside of the Forest Stewardship Plan

Archeology and TUS

| Archeological Potential | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development overlaps with high arch potential polygons. | X |  |
| Comments: As per the Provincial data (RAAD), all blocks except 160 and177 overlap with a “high archaeological” value polygon. |  |  |
| Traditional Use Data | APPLIES | DOES NOT APPLY |
| Proposed development overlaps or is adjacent to traditional use sites. |  |  |
| Comments (type of site, proximity): |  |  |
| Remote Access to Archaeological Data (RAAD) | APPLIES | DOES NOT APPLY |
| Proposed development overlaps or is adjacent to known arch sites in the RAAD database. |  | X |
| Comments (type of site, proximity): As per the Provincial data (RAAD), the proposed development blocks do not overlap or are adjacent to the known traditional use sites (TUS). |  |  |

Tsilhqot’in National Government letter dated October 2, 2013*)

| Tsilhqot’in National Government (Request for questions to be addressed within future TNG referrals.) | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is referred to Tsilhqot’in National Government or Yunesit’in Government. |  |  |
| Management of thermal cover for moose. |  |  |
| Management of moose vulnerability through security cover. |  |  |
| Management of moose vulnerability through access control. |  |  |
| Comments: TNG Zone A Consultative Area |  |  |

*)Link to TNG letter: G:\BCTS\Planning\!Referral - First Nations & Stakeholders\FN Consultation General\TNG

# Migratory Birds Convention Act (1994)

| Migratory Bird Nesting Habitat | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development adjacent to or overlapping a stand with the following migratory bird nesting habitat ranking of: | X |  |
| Comments: Block; Rank. 153; 4&5. 155; 5. 160; 4. 162; 4. 176; 4. 177; 4. 178; 4&5. 180; 4. A field assessment needed. |

# Conservation Legacy Area*)

| Conservation Legacy Area | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Conservation Priority Rating and FSP 5.11.6 Results and Strategies: 6 |  | X |
| Comments: N/A |  |  |

*) “Quesnel Forest District Enhance Retention Strategy – for Large Scale Salvage of Mountain Pine Beetle Impacted Stands”,

Released 10, February 2006.

Link: https: //www.for.gov.bc.ca/ftp/dqu/external/!publish/matrix/PDF/DQU%20Enhanced%20Conservation%20Strategy-%20Release%201.pdf

# TCC Landscape Connectivity Corridor

| TCC Landscape Connectivity Corridor | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development within or adjacent to a landscape connectivity corridor. | X |  |
| Comments: See Section 5.11.2 |  |  |

# Terrain Stability Class

| Terrain Stability Class | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Review terrain stability overview data. Does the proposed block overlap terrain stability class 3, 4 or 5? | X |  |
| Comments: See Section 5.8 |  |  |

# Baker Creek Watershed & Other of Interest Watersheds*)

| Baker Creek Watershed | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development is within Baker Creek watershed |  | X |
| Comments (describe sub-basin, aspect, elevation, above or below H60 line, conservation priority): N/A |  |  |

*) Link to the 2010 Report Recommendations: G:\BCTS\Planning\Baker Creek Watershed\QU district final report Feb 2011\Baker Creek report recommendations follow up 2010[1].pdf

Permanent Sample Plots and Government Forest Research Installations

| Permanent Sample Plots and Government Forest Research Installations * | APPLIES | DOES NOT APPLY |
| --- | --- | --- |
| Proposed development adjacent to or overlapping known permanent sample plot or known forest research installation location. |  | X |
| Comments: N/A Spatial location of known permanent sample plots and forest research installations can be found on the peaches drive at the following location: tcc_root\General_User_Workspace\TCC PLANNING\ResearchPlots Guidance documents for overlap with permanent sample plot locations can be found on the G drive at the following location: G:\BCTS\Planning\Permanent Sample Plot Guidance |  |  |

# Stakeholders Status Check

Status Check to be Reviewed

Guide: All blocks overlap guide tenure Iverson, Brian; 500954. Trapper: All blocks overlap trapper tenure TR0515T020. Range: No overlaps Grazing Lease: No overlaps Recreation Clubs: No overlaps Tourism Operations: All blocks overlap two commercial recreation licences. - Crown Land 5406691-Guided Nature Viewing - Crown Land 5406975 -Multiple Use Licenced Waterworks: No overlaps Mineral: Company; Tenure no.; Block; Type. SLUCHINSKI, EVAN REX; 508428; 155; Placer Cell Title Submission. ASHTON, THOMAS ANDREW; 1056797; 155; Placer Cell Title Submission. BZ MINERAL RESOURCES LTD.; 596986; 155; Placer Cell Title Submission. TITOSKY, DOUGLAS BRENT; 1023111; 155; Placer Cell Title Submission. SLUCHINSKI, EVAN REX; 1013880; 155; Placer Cell Title Submission. BELL, JOHN MALCOLM; 1077875; 153&155; Mineral Cell Title Submission. SLAUTIN, OLEKSANDR; 1028929; 153&155; Placer Cell Title Submission. KARUS GOLD CORP.; 519042; 153; Mineral Cell Title Submission. MANKE, DONALD GERALD; 1076540; 153; Placer Cell Title Submission. BREWER, JAMES DALE; 516594; 153; Placer Cell Title Submission. ZION MINERALS LTD; 1064689; 153; Mineral Cell Title Submission. MANKE, DONALD GERALD; 1083110; 153; Placer Cell Title Submission. ZION MINERALS LTD; 1065027; 153; Mineral Cell Title Submission. MANKE, DONALD GERALD; 1083112; 153; Placer Cell Title Submission. BARKER MINERALS LTD; 1070173; 160, 162, 177, 178, 180; Mineral Cell Title Submission. Private Land: No overlaps Road and Blocks: Block; Tenure Type. FTEN Cutblock Poly. Forest Road Section R02156 24. Forest Cut Permits A20017 556.

The Planning Forester and Practices Forester have reviewed and are aware of all information in this document:

Planning Forester:									Date:

Print Name & Sign, RPF

Practices Forester:									Date:

Print Name & Sign, RPF

Map Attachments:

☐	Moose Thermal

☐	Moose Security

☐	Moose Wetland Shape Comparison

☒	Fisher

☐	Furbearers Riparian Shape Comparison

☐	Riparian Shape Comparison

☐	LMZ Disturbance

☐	Grassland Benchmark Shape Comparison

☒	Mature Birch

☐	Viewshed

☒	Buffered Trails

☐	Pine Mushroom

☐	CASC

☐	Invasive Plants