The purpose of this checklist is to confirm that the required steps in the timber sale planning process as they relate to safety have been considered and integrated into the licence document or tender.

| Date: June 24, 2021 |  |  |
| --- | --- | --- |
| Timber Sale Licence #: TA0904 Blk 1 |  |  |
| BCTS Representative: |  |  |
| Safety Considerations = Applicable = Not applicable | Comments |  |
| Any Known Hazards (on-site / adjacent) Overhead hazards (snags/power lines) Previous activities such as road building have not created hazards such as unstable fill slopes, rocks or log decks that will effect harvesting? Steep Slopes (+35% for ground based harvesting) information to be provided to prospective licensees to support their harvest planning such as: - Slopes that exceed 35% - Long pitches of continuous slope - Terrain Stability Features e.g. fracture rock formation - Ground Roughness e.g. hummocks, depressions steep, ravines - Soil Conditions e.g. soil depth, exposed rock, loose gravels or soft mud. - Timber size and quality - Slopes or benches less than 35% but located above areas of greater slope. Dangerous Terrain (gullies, ridges, cliffs) Identified Terrain Assessment safety considerations Unstable Surfaces (wet, soft) Wind throw / Heavy Slash Road Hazards (Road grades exceeding 18%). Road Excavation hazard (road sections in terrain >33% + anticipated cutbanks >6m in height + any other area that shows sign of instability). Road Construction Initiated Landslides Hazards (signs of instability, broken rock, transition zones). Wildlife Mines / Electrical / Pipelines Other Risks / Hazards on site / adjacent / Root rot centres or infection levels | Overhead hazards: This block has been damaged by wildfire of varying severity. In areas of greater burn severity, tree roots have been burnt out rendering trees highly unstable. There is a high hazard of sudden tree fall. Workers are to stay in machinery when working in such areas. Do not take breaks out of equipment in highly damaged areas. Monitor winds as risk of tree fall increases with increasing wind speeds. In areas of greater burn severity, trees may be structurally compromised and may break suddenly creating widowmakers that pose high hazard to workers. Steep Slopes: Short pitches of >35% occur in the block.. Prior to any machinery working on slopes exceeding the manufacturer recommended limit or if not available (>35%), a steep slope hazard assessment must be completed and documented by qualified personel. Boulders: Large boulders may present a rolling hazard to workers on steeper terrain. Heavy Windthrow: Areas of heavy windthrow occur in the block as result of fire damage. An area of concern is noted on the harvest plan map indicating an area of severe blowdown, Machinery must work carefully in this area as there may be unknown watercourses underneath. Trees may be under compression and may violently release when pressure is removed. Windthrow is present throughout the block as result of root damage resulting from the fire. Road Hazards: Danger trees occur along all access routes for this block. Danger trees are to be removed where they can strike landings, decking areas. Prior to operations, the harvesting supervisor should check access routes for downed trees blocking roads and ensure they are cleared. At the start of the Bear West FSR, a steep grade with narrow width occurs. While this pitch is accessible for logging trucks, drivers must exercise extreme caution when coming down loaded as there is a tight corner with steep dropoff. This should not be driven in icy or slippery muddy conditions. The Bear Main accesses the Bear Lake Recreation site. Public traffic is possible. Signage indicating that active falling and hauling are taking place are to be posted at the start of the Bear Main FSR and at the entry of the work area. Wildlife: Wildlife may frequent burns for foraging regrowth. Other: Large boulders are present in the block. These may pose obstacles to equipment and pose a rolling hazard. |  |
| Term Adequate time to complete work given seasonal and access considerations? | Harvest during frozen conditions or on snowpack preferred. Otherwise harvest to be carried out during dry conditions that will minimize risk of soil disturbance. |  |
| Access Has access route been assessed for hazards? Is their a hazard mitigation strategy in place that will be enacted prior to worker exposure? Will access routes accommodate the nature of traffic? (See Checklist 23-1 for more comprehensive guidance) | Harvest supervisor to check access route for downed trees prior to moving in equipment. Many danger trees occur along access routes to this block. Supervisor to determine mitigation requirements and ensure hazards to equipment and workers are removed. Danger trees within reach of landing and decking areas must be removed. Basic upgrades and maintenance wil be needed as indicated in Road site plan. |  |
| Safety Considerations = Applicable = Not applicable | COMMENTS |  |
| Adjacent Operations Are there conflicts with adjacent operations relative to timing or hazards (major licensees, salvage, silviculture, other industrial or public operations, phase congestion)? | No conflicts anticipated. |  |
| Site Plan and harvest systems: Have the proposed plans been reviewed with consideration toward worker safety? Have all known hazards been documented (high retention, wind throw, snags, unstable slopes, steeps slopes)? | Known safety hazards have been identified, and where applicable indicated on maps. Slopes >35% are indicated on the harvest plan maps. Area of concern (severe windthrow) is noted on HP map. |  |
| Essential Elements |  | Comments |
| Other Factors: |  |  |
| Follow-up Action Required: |  |  |